'Program to read the SMT160 with the 2313 AVR

$include 2313def.inc

$crystal = 1000000

Declare Sub Readsmt

Declare Sub Intubrr

Dim T1 As Word

Dim T2 As Word

Dim T3 As Word

Dim Dc As Single

Dim Tem As Word

Dim Tem1 As Word

Dim Tem2 As Word

Dim Minus As Bit

Open "Comd.1 : 19200 , 8 , N , 1"                          ' For Output As #1

rcall intubrr

Do

rcall readsmt

T3 = T1 + T2

Dc = T1 / T3

Dc = Dc - 0.31924

Dc = Dc / 0.00472

Dc = Dc * 100

Tem = Dc

Reset Minus

If Tem > 32767 Then

Set Minus

Tem = 65536 - Tem

End If

Tem1 = Tem / 100

Tem2 = Tem1 * 100

Tem2 = Tem - Tem2

Print "1: " ;

If Minus = 1 Then

Print "-" ;

Else

Print " ";

End If

Print Tem1 ; "." ;

If Tem2 < 10 Then Print "0";

Print Tem2

'   Wait 10

'   Wait 10

Loop

Sub Readsmt

T1 = 0

$asm

.def T1ll = R7

.def T1lh = R8

.def T1hl = R9

.def T1hh = R10

.def T2ll = R11

.def T2lh = R12

.def T2hl = R13

.def T2hh = R14

.def One = R16

.def Zero = R17

.def Temp = R18

.def Tempin = R18

.def Tempr = R19

.def Cntin = R20

.def T1lt = R25

.def T1ht = R26

.def T2lt = R27

.def T2ht = R28

.def Mescnt = R29

.equ T1port = Pinb

.equ T1pin = 0

.equ Timer1_clock = 65500

ldi one,1

ldi zero,0

clr t1ll

clr t1lh

clr t1hl

clr t1hh

clr t2ll

clr t2lh

clr t2hl

clr t2hh

Main:

ldi cntin,2

ldi mescnt,128

Out Tcnt1h , Zero

Out Tcnt1l , Zero

Out Tccr1b , One

Main10:

sbic t1port,t1pin

rjmp main10

Main11:

sbis t1port,t1pin

rjmp main11

Out Tccr1b , One

Out Tcnt1h , Zero

Out Tcnt1l , Zero

Main12:

sbic t1port,t1pin

rjmp main12

Out Tccr1b , Zero

in t1lt,tcnt1l

in t1ht,tcnt1h

Out Tcnt1h , Zero

Out Tcnt1l , Zero

Main13:

sbis t1port,t1pin

rjmp main13

Main14:

sbic t1port,t1pin

rjmp main14

Out Tccr1b , One

Main15:

sbis t1port,t1pin

rjmp main15

Out Tccr1b , Zero

in t2lt,tcnt1l

in t2ht,tcnt1h

Out Tcnt1h , Zero

Out Tcnt1l , Zero

add   t1ll,t1lt

adc   t1lh,t1ht

adc   t1hl,zero

adc   t1hh,zero

add   t2ll,t2lt

adc   t2lh,t2ht

adc   t2hl,zero

adc   t2hh,zero

dec mescnt

brne main10

ldi mescnt,128

dec  cntin

brne main10

cli

rjmp main17

Main16:

clr t1ll

clr t1lh

clr t1hl

clr t1hh

clr t2ll

clr t2lh

clr t2hl

clr t2hh

Out Tccr1b , Zero

Main17:

cli

Loadadr T1 , X

st    x+,t1lh

st    x,t1hl

Loadadr T2 , X

st    x+,t2lh

st    x,t2hl

Return

$end Asm

End Sub

Sub Intubrr

$asm

Ldi r16, 64

Out Ubrr , R16

$end Asm

Return

End Sub