# Steps how to flash a GPS FW onto the NAVI-XT

- Start up the NAVI-XT

- Create a BlueTooth connection between  the NAVI-XT and the PC

- The Bluetooth connection occupies a virtual Port

- Start up the SiRF-Flash Tool 3.1 or higher (see photo)

- In  SiRF flash tool Select the correct port  which is occupied by the NAVI-XT

- Important: Change the baudrate in SiRF-Flash Tool  to 38400 (see photo). If you you leave it at 115200 the unit could be damaged..

- Select „Erase whole chip“ (see photo)

- Select „External“ (see photo)

- Hit the button „Browse“ to select the FW you want to flash

- Hit the button „Execute“to start flashing

- The Flash-Process can be seen in the Progress-Bars

- After a successful  flashing, reset the NAVI-XT