Step 1:

pal_boardtype.h :

/* ZigBit ZDM_1281_A2 with AT86RF230B Radio Transceiver */

#define ZIGBIT_ZDM_A1281_A2		(0x16)

Step2:

pal_irq.c:

#if (BOARD_TYPE == ZIGBIT_ZDM_A1281_A2)

#endif /* ZIGBIT_ZDM_A1281_A2 */

Step3:

pal.config.h:

#include "pal_boardtypes.h"

#if (BOARD_TYPE == ZIGBIT_ZDM_A1281_A2)

/* Enumerations used to idenfify LEDs */

typedef enum led_id_tag

{

LED_0,

LED_1,

LED_2

} led_id_t;

#define NO_OF_LEDS                      (3)

// Buttons habe ich nicht auf meiner Platine, deshalb habe ich hier weggemacht und auch überall wo //aufgerüft wird

/* === Externals ============================================================*/

/* === Macros ===============================================================*/

/** The default CPU clock */

#ifndef F_CPU

#define F_CPU                   (8000000UL)

#endif

/*

* IRQ macros for ATmega1281

*/

/*

* AT86RF230B:

*

* TRX_MAIN_IRQ_HDLR_IDX       TRX interrupt mapped to MCU input capture pin

* TRX_TSTAMP_IRQ_HDLR_IDX       Not used

*/

/* Number of used TRX IRQs in this implementation */

#define NO_OF_TRX_IRQS                  (1)

/* Mapping of TRX IRQs to MCU pins */

#define TRX_IRQ                         (TIMER1_CAPT_vect)

/** Enables the transceiver interrupts. */

#define ENABLE_TRX_IRQ(trx_irq_num)     do {    \

if (trx_irq_num == TRX_MAIN_IRQ_HDLR_IDX)          \

{                                           \

/* Enable RX TIME STAMP interrupt */    \

TIMSK1 |= _BV(ICIE1);                   \

}                                           \

} while (0)

/** Disables the transceiver interrupts. */

#define DISABLE_TRX_IRQ(trx_irq_num)     do {   \

if (trx_irq_num == TRX_MAIN_IRQ_HDLR_IDX)          \

{                                           \

/* Disable RX TIME STAMP interrupt */   \

TIMSK1 &= ~(_BV(ICIE1));                \

}                                           \

} while (0)

/** Clears the transceiver interrupts. */

#define CLEAR_TRX_IRQ(trx_irq_num)     do {     \

if (trx_irq_num == TRX_MAIN_IRQ_HDLR_IDX)          \

{                                           \

/* Clear RX TIME STAMP interrupt */     \

TIFR1 = _BV(ICF1);                      \

}                                           \

} while (0)

/** Enables the global interrupts. */

#define ENABLE_GLOBAL_IRQ()             sei()

/** Disables the global interrupts. */

#define DISABLE_GLOBAL_IRQ()            cli()

/*

* This macro saves the global interrupt status

*/

#define ENTER_CRITICAL_REGION()         {uint8_t sreg = SREG; cli()

/*

*  This macro restores the global interrupt status

*/

#define LEAVE_CRITICAL_REGION()         SREG = sreg;}

/*

* This macro saves the trx interrupt status and disables the trx interrupt.

*/

#define ENTER_TRX_REGION()      { uint8_t irq_mask = TIMSK1; TIMSK1 &= ~(_BV(ICIE1))

/*

*  This macro restores the transceiver interrupt status

*/

#define LEAVE_TRX_REGION()      TIMSK1 = irq_mask; }

/*

* GPIO macros for ATmega1281

*/

/*

* This board uses an SPI-attached transceiver.

*/

#define PAL_USE_SPI_TRX                 (1)

/* Actual Ports */

/*

* PortB DDR is data direction register for the transceiver

*/

#define TRX_PORT1_DDR                   (DDRB)

#define TRX_PORT2_DDR				    (DDRA)

/*

* PORTB is transceiver port

*/

#define TRX_PORT1                       (PORTB)

#define TRX_PORT2				   (PORTA)

/*

* Slave select pin is PORTB 0

*/

#define SEL                             (PB0)

/*

* serial clock pin is PORTB 1

*/

#define SCK                             (PB1)

/*

* SPI Bus Master Output/Slave Input pin is PORTB 2

*/

#define MOSI                            (PB2)

/*

* SPI Bus Master Input/Slave Output pin is PORTB 3

*/

#define MISO                            (PB3)

/*

* Sleep Transceiver pin is PORTB 4

*/

#define SLP_TR                          (PB4)

/*

* RESET pin is PORTA 7

*/

#define RST                             (PA7)

/*

* Set TRX GPIO pins.

*/

#define RST_HIGH()                      (TRX_PORT2 |= _BV(RST))

#define RST_LOW()                       (TRX_PORT2 &= ~_BV(RST))

#define SLP_TR_HIGH()                   (TRX_PORT1 |= _BV(SLP_TR))

#define SLP_TR_LOW()                    (TRX_PORT1 &= ~_BV(SLP_TR))

/*

* PORT where LEDs are connected

*/

#define LED_PORT                        (PORTB)

#define LED_PORT_DIR                    (DDRB)

/*

* PINs where LEDs are connected

*/

#define LED_PIN_0                       (PB5)

#define LED_PIN_1                       (PB6)

#define LED_PIN_2                       (PB7)

/*

* Value of an external PA gain.

* If no external PA is available, the value is 0.

*/

#define EXTERN_PA_GAIN                  (0)

/*

* Timer macros for ATmega1281

*/

/*

* This value is used by the calibration routine as target value of the timer

* to be calibrated. At the end of a calibration cycle this value is compared

* with the counter result.

* TARGETVAL_CALIBRATION = CLK(Cal) * Count(Ref) / CLK(Ref) = 8,000,000 * 256 / 32,768

*

* CLK(Cal): Frequency of clock source of timer to be calibrated

* CLK(Ref): Frequency of clock source of reference timer

* Count(Ref): Value of reference timer after 1 full cycle (until overflow)

*             for timer width of 8 bit

*/

#if (F_CPU == (8000000UL))

#define TARGETVAL_CALIBRATION           (62500)

#elif  (F_CPU == (4000000UL))

#define TARGETVAL_CALIBRATION           (31250)

#elif  (F_CPU == (2000000UL))

#define TARGETVAL_CALIBRATION           (15625)

#elif  (F_CPU == (1000000UL))

#define TARGETVAL_CALIBRATION           (7813)

#else

#error "Unsupported F_CPU value"

#endif

/** This value is the loop counter for calibration routine. */

#define MAX_CAL_LOOP_CNT                (100)

/*

* These macros are placeholders for delay functions for high speed processors.

*

* The following delay are not reasonbly implemented via delay functions,

* but rather via a certain number of NOP operations.

* The actual number of NOPs for each delay is fully MCU and frequency

* dependent, so it needs to be updated for each board configuration.

*

* ATmega1281 @ 8MHz

*/

/*

* Wait for 65 ns.

* time t7: SLP_TR time (see data sheet or SWPM).

* In case the system clock is slower than 32 MHz we do not need

* to have this value filled.

*/

#define PAL_WAIT_65_NS()                // empty

/* Wait for 500 ns. */

#define PAL_WAIT_500_NS()               {nop(); nop(); nop(); nop(); }

/* Wait for 1 us. */

#define PAL_WAIT_1_US()                 {nop(); nop(); nop(); nop(); nop(); nop(); nop(); nop();}

/*

* The smallest timeout in microseconds

*/

#define MIN_TIMEOUT                     (0x80)

/*

* The largest timeout in microseconds

*/

#define MAX_TIMEOUT                     (0x7FFFFFFF)

/**

* Minimum time in microseconds, accepted as a delay request

*/

#define MIN_DELAY_VAL                   (5)

/*

* Timer clock source while radio is awake.

*/

#define TIMER_SRC_DURING_TRX_AWAKE() \

(TCCR1B = ((1 << ICES1) | (1 << CS12) | (1 << CS11) | (1 << CS10)))

/*

* Timer clock source while radio is sleeping.

*/

#define TIMER_SRC_DURING_TRX_SLEEP() \

(TCCR1B = (1 << ICES1) | (1 << CS11))

/*

* Maximum numbers of software timers running at a time

*/

#define MAX_NO_OF_TIMERS (25)

#if (MAX_NO_OF_TIMERS > 255)

#error "MAX_NO_OF_TIMERS must not be larger than 255"

#endif

/*

* Hardware register that holds Rx timestamp

*/

#define TIME_STAMP_REGISTER             (ICR1)

/*

* TRX Access macros for ATmega1281

*/

/*

* Bit mask for slave select

*/

#define SS_BIT_MASK                     (1 << SEL)

/*

* Slave select made low

*/

#define SS_LOW()                        ((TRX_PORT1) &= (~SS_BIT_MASK))

/*

* Slave select made high

*/

#define SS_HIGH()                       ((TRX_PORT1) |= (SS_BIT_MASK))

/*

* Mask for SPIF bit in SPSR register

*/

#define SPIF_MASK                       (1 << SPIF)

/*

* Wait for SPI interrupt flag

*/

#define SPI_WAIT()                      while (!(SPSR & SPIF_MASK))

/*

* Dummy value written in SPDR to retrieve data form it

*/

#define SPI_DUMMY_VALUE                 (0x00)

/*

* TRX Initialization

*/

#define TRX_INIT()                      do {                        \

/* Enable the SPI and make the microcontroller as SPI master */ \

SPCR = _BV(SPE) | _BV(MSTR);                                    \

SPSR = _BV(SPI2X); /* Double the SPI clock frequency */         \

/* Select the serial clock SCK to be (FOSC/4)                   \

* For Example for ATmega1281, FOSC = 8Mhz                      \

* (Microcontroller operating frequency)                        \

* SPI clock is thus set to (8/4)*2 = 4 Mhz                     \

*/                                                             \

SPCR &= ~(_BV(SPR0) | _BV(SPR1));                               \

\

/* Set SEL pin which is on PortB to high */                     \

TRX_PORT1 |= _BV(SEL);  									   \

TRX_PORT2 |= _BV(SEL);                                         \

} while (0)

/*

* SPI Interrupt enable macro

*/

#define SPI_IRQ_ENABLE()    SPCR |= _BV(SPIE)

/*

* SPI Interrupt disable macro

*/

#define SPI_IRQ_DISABLE()    SPCR &= ~_BV(SPIE)

/*

* SPI Interrupt clear macro

*/

#define SPI_IRQ_CLEAR()     SPSR &= ~_BV(SPIF)

/*

* UART0 and UART1 non-generic (board specific) initialization part.

*/

/* Data Length is 8 bit */

#define UART_0_INIT_NON_GENERIC()   (UCSR0C = (1 << UCSZ01) | (1 << UCSZ00))

#define UART_1_INIT_NON_GENERIC()   (UCSR1C = (1 << UCSZ11) | (1 << UCSZ10))

/*

* This board does NOT have an external eeprom available.

*/

#ifndef EXTERN_EEPROM_AVAILABLE

#define EXTERN_EEPROM_AVAILABLE            (0)

#endif

/*

* Alert initialization

*/

#define ALERT_INIT()                    do {    \

LED_PORT = 0;                           \

LED_PORT_DIR = 0xFF;                    \

} while (0)

/*

* Alert indication

*/

#define ALERT_INDICATE()                (LED_PORT = (uint8_t)(~(uint16_t)(LED_PORT)))

/* === Prototypes ===========================================================*/

#ifdef __cplusplus

extern "C" {

#endif

#ifdef __cplusplus

} /* extern "C" */

#endif

/*

* MCU pins used for debugging

*/

#ifdef ENABLE_DEBUG_PINS		// muss hier was ändern?

// Pin and port assignment

#define TST_PIN_0_HIGH()                PORTF |= (1 << 0)

#define TST_PIN_0_LOW()                 PORTF &= ~(1 << 0)

#define TST_PIN_1_HIGH()                PORTF |= (1 << 1)

#define TST_PIN_1_LOW()                 PORTF &= ~(1 << 1)

#define TST_PIN_2_HIGH()                PORTF |= (1 << 2)

#define TST_PIN_2_LOW()                 PORTF &= ~(1 << 2)

#define TST_PIN_3_HIGH()                PORTF |= (1 << 3)

#define TST_PIN_3_LOW()                 PORTF &= ~(1 << 3)

#define TST_PIN_4_HIGH()                PORTE |= (1 << 2)

#define TST_PIN_4_LOW()                 PORTE &= ~(1 << 2)

#define TST_PIN_5_HIGH()                PORTE |= (1 << 3)

#define TST_PIN_5_LOW()                 PORTE &= ~(1 << 3)

#define TST_PIN_6_HIGH()                PORTE |= (1 << 4)

#define TST_PIN_6_LOW()                 PORTE &= ~(1 << 4)

#define TST_PIN_7_HIGH()                PORTE |= (1 << 6)

#define TST_PIN_7_LOW()                 PORTE &= ~(1 << 6)

#define TST_PIN_8_HIGH()                PORTE |= (1 << 7)

#define TST_PIN_8_LOW()                 PORTE &= ~(1 << 7)

#else   /* ENABLE_DEBUG_PINS */

// empty pin assignment

#define TST_PIN_0_HIGH()

#define TST_PIN_0_LOW()

#define TST_PIN_1_HIGH()

#define TST_PIN_1_LOW()

#define TST_PIN_2_HIGH()

#define TST_PIN_2_LOW()

#define TST_PIN_3_HIGH()

#define TST_PIN_3_LOW()

#define TST_PIN_4_HIGH()

#define TST_PIN_4_LOW()

#define TST_PIN_5_HIGH()

#define TST_PIN_5_LOW()

#define TST_PIN_6_HIGH()

#define TST_PIN_6_LOW()

#define TST_PIN_7_HIGH()

#define TST_PIN_7_LOW()

#define TST_PIN_8_HIGH()

#define TST_PIN_8_LOW()

#endif /* ENABLE_DEBUG_PINS */

#endif /* ZIGBIT_ZDM_A1281_A2 */

#endif  /* PAL_CONFIG_H */

/* EOF */

Step 4:

pal_board.c:

#include "pal_timer.h"

#if (BOARD_TYPE == ZIGBIT_ZDM_A1281_A2)

/**

* @brief Initializes the GPIO pins

*

* This function is used to initialize the port pins used to connect

* the microcontroller to transceiver.

*/

void gpio_init(void)

{

/* The following pins are output pins.  */

TRX_PORT1_DDR |= _BV(SEL);

TRX_PORT1_DDR |= _BV(SCK);

TRX_PORT2_DDR |= _BV(RST);

TRX_PORT1_DDR |= _BV(MOSI);

TRX_PORT1_DDR |= _BV(SLP_TR);

/* The following pins are input pins.  */

TRX_PORT1_DDR &= ~_BV(MISO);

}

/**

* @brief Initialize LEDs

*/

void pal_led_init(void)

{

/* Entire port is used for LEDs. */

LED_PORT = 0xE0;

LED_PORT_DIR = 0xE0;

}

/**

* @brief Generation of random seed for function srand() in case this

*        is not supported by the transceiver (e.g. AT86RF230)

*

* @return uint16_t Random number to be used as seed for function srand()

* @ingroup apiPalApi

*/

uint16_t pal_generate_rand_seed(void)

{

/* Todo */

return (0);

}

#endif /* ZIGBIT_ZDM_A1281_A2*/

/* EOF */

Step5:

main.c // Star_Nobeacon

/* === MACROS ============================================================== */

#ifdef CHANNEL

#define DEFAULT_CHANNEL                 (CHANNEL)

#define DEFAULT_CHANNEL_PAGE            (0)

#else

/** Defines the default channel. */

#if (TAL_TYPE == AT86RF230B)

#ifdef CHINESE_BAND

#define DEFAULT_CHANNEL                 (0)

#define DEFAULT_CHANNEL_PAGE            (5)

#else

#define DEFAULT_CHANNEL                 (1)

#define DEFAULT_CHANNEL_PAGE            (0)

#endif  /* #ifdef CHINESE_BAND */

#else

#define DEFAULT_CHANNEL                 (20)

#define DEFAULT_CHANNEL_PAGE            (0)

#endif  /* #if (TAL_TYPE == AT86RF230B) */

#endif  /* #ifdef CHANNEL */

/** Defines the PAN ID of the network. */

#ifdef PAN_ID

#define DEFAULT_PAN_ID                  (PAN_ID)

#else

#define DEFAULT_PAN_ID                  (0xBABE)

#endif

/** Defines the short address of the coordinator. */

#define COORD_SHORT_ADDR                (0x0000)

/** Defines the maximum number of devices the coordinator will handle. */

#define MAX_NUMBER_OF_DEVICES           (3)

/** This is the time period in micro seconds for data transmissions. */

#define DATA_TX_PERIOD                  (2000000)

/** Defines the bit mask of channels that should be scanned. */

#if (TAL_TYPE == AT86RF230B)

#ifdef CHINESE_BAND

#define SCAN_ALL_CHANNELS           (0x0000000F)

#else

#if (DEFAULT_CHANNEL == 0)

#define SCAN_ALL_CHANNELS       (0x00000001)

#else

#define SCAN_ALL_CHANNELS       (0x000007FE)

#endif

#endif  /* #ifdef CHINESE_BAND */

#else

#define SCAN_ALL_CHANNELS               (0x07FFF800)

#endif

Step 6:

makefile:

_TAL_TYPE = AT86RF230B

_PAL_TYPE = ATMEGA1281

_PAL_GENERIC_TYPE = AVR

_BOARD_TYPE = ZIGBIT_ZDM_A1281_A2

_HIGHEST_STACK_LAYER = MAC

Step 7:

Makefile_debug:

# Build specific properties

_TAL_TYPE = AT86RF230B

_PAL_TYPE = ATMEGA1281

_PAL_GENERIC_TYPE = AVR

_BOARD_TYPE = ZIGBIT_ZDM_A1281_A2

_HIGHEST_STACK_LAYER = MAC

Step 8:

Star.ewp: Überall stat REB_2_3_STK500_STK501 habe ZIGBIT_ZDM_A1281_A2 eingegeben

Step9:

Star.aps:

<SOURCEFILE>..\..\..\..\PAL\AVR\ATMEGA1281\Boards\ZIGBIT_ZDM_A1281_A2\pal_irq.c</SOURCEFILE>

<SOURCEFILE>..\..\..\..\PAL\AVR\ATMEGA1281\Boards\ZIGBIT_ZDM_A1281_A2\pal_board.c</SOURCEFILE>

Step10:

EEPROM Addresse habe ich so eingegeben, wie es in User Guide beschrieben ist.

Result: alle drei LED blinkelt gleichzeitigt, dass eigentlich falsh sein sollte.

Verwendete Application: Star_Nobeacon