I/O-Liste

Schnittstelle Teleskop		COM 1, 9600 Baud, No Parity, 8 Data, 1 Stop

Schnittstelle Motorregler		COM 2, 19200 Baud, No Parity, 8 Data, 1 Stop

Schnittstelle Dig Output PC	LPT1, Bit 7	Power on / off

LPT1, Bit 8	Manuel / Auto

Befehle Teleskop:

:hP#		Park Position				no return

:hN#		Sleep Telescope			no return

:hW#		Wake Up Telescope		no return

:GZ#		Get Telescop Azimut		DDD*MM#	(z.B. 360:00)

Befehle Motorregler:

pm		Position Mode			echo

st		End Mode, Drive off			echo

ma n		Move Absolute to n			echo

rp		Read Drive Position			echo

ca 4		Calibration Drive			echo

sp 0		Set Drive Counter Zero		echo

ss 1-8		Status System			n = 1/2/4/8/16/32/64/128/256

0 = low

n = high (true)

Bit 3 high = Auto Mode

Bit 4 low   = Drive off		Bit 4 high = Drive in Motion

Bit 6 high = Drive calibrated

Bedienelemente:

S1	Switch		Power On / Off			(und wenn LPT1, Bit 8 = high)

L1	Light	 	Rot    = Power off

Grün = Power on

S3	Switch		Manuel / Auto			(und wenn LPT1, Bit 7 = high)

Auto =		Open COM 1

Open COM2

L2	Light		Rot   = Manuel

Grün = Auto

L3	Light		Drive calibrated = Grün

LB1	Listbox	Telescope AZ-Position		3-stellig num. (0-360) Grad

LB2	Listbox	Dome AZ-Position act.		3-stellig num. (0-360) Grad

B1	Buton		Manuel Mode Left			wenn S2 = Manuel

B2	Buton		Manuel Mode Right			wenn S2 = Manuel

L4	Light		Dome in Motion			wenn AZ-Dome <> AZ-Telesc.

und ss 4 = 16 (high)

L5	Light		Dome in Position			wenn AZ-Dome = AZ-Telesc.

und ss 4 = 0 (low)

T1	Textbox	Command Dome			20-stellig char.

Schreiben COM2			Aktiv wenn S2 Manuel

LB2	Listbox	Echo Dome				20 stellig char.

Echo COM2				immer aktiv

T2	Textbox	Command Telescope		20-stellig char.

Schreiben COM1			Aktive wenn S2 Manuel

LB3	Listbox	Echo Dome				20-stellig char.

immer aktiv

B3	Buton		Reset COM1				close COM1, open COM1

B4	Buton		Reset COM2				close COM2, open COM2

LB4	Listbox	Staus Message				20-stell char.

Algorithmus:

Wenn S2 = Auto, dann:

Kalibrierung Dome: comand ‚ca 4’ an COM2

If ss 6 = 64 (high, Drive kalibirert) dann sp 0 (drive counter reset to zero) und L3 = on

Lese Drive Position rp, if rp = 0 dann Drive okay if <> 0 Drive dann LB4 ‘Drive Error’

If Drive okay dann lesen Azimut Teleskop COM1 mit :GZ#,  im 2 sec. Zyklus

Echo z.B. 265:15#	(265 Grad, 15 Minuten)

Def. Variable Teleskop Azimut Position ‚az_scope’ = 3 num. von links  265

Def. Encoder Max. Wert: ‚enc_max’ = 85.500	(Impulse für 360° Dome Rotation)

Def. Variable Dome Soll Position ‘dome_pos1’

Umrechnung :

‚az_scope’ 	=	0°		‘dome_pos1’	=	0

‚az_scope’ 	=	360°		‘dome_pos1’	=	encmax (= 85.500)

Fahre Dome nach ‘dome_pos’ comand ma n (n = wert 0 – 85.500 aus Umrechnung)

Variable Dome Ist Position ‘dome_act’ (comand ‚rp’) auslesen, zurückrechnen in volle Grad (0-360) und anzeigen in LB2

Wenn ‚dome_pos1’ <> ‚dome_act’ und ss 4 = 16 (high) dann L3 (anzeigen Dome in Motion)

Wenn ‚dome_pos1 = ‚dome_act’ und ss 4 = 0 (low) dann L4 (anzeigen Dome in Position)

Wenn ss 4 = 16 (high) dann timer start

Wenn timer = 120 (sec) dann st (drive Stop) und Ausgabe LB4 = Position Time Error

Beispiel: Interface

Dome Control System

Power on/off

Manual / Auto

Drive calibrated

Drive in Motion Motionrated

Drive in Position Stopcalibrated

Telescope AZ Position PPosition AZDrive in Position Stopcalibrated

265°

Dome AZ Position PPosition AZDrive in Position Stopcalibrated

265°

Reset COM1

Reset COM2

Telescope Command Position PPosition AZDrive in Position Stopcalibrated

:hP#

Telescope Echo Position PPosition AZDrive in Position Stopcalibrated

---

Drive Command

C

Telescope Command Position PPosition AZDrive in Position Stopcalibrated

PM

Drive Echo

PM

Error Message

COM1 - no connection -