MDFS 3.39/4.8/6.1s

MonitoringData FileSystem 3InfoBits, 39Data1/4SpecialData1 Bits, 8Long/6Short Data2 Bits

Code v.1 with “Special Use” function

Monitoring 39Bit Decoded Time an 9Bit Temperature with DataEnd Detection

InfoBits:

0:	Time / Data	/Special Use				1:	Temperature

x:	Refreshrate0	/Special Use			0: Long Code 8Bit	1: Short Code 6Bit

x:	Refreshrate1	/Special Use				x:	Signed

“Special Use” marked by wrong Data in Bit 4-7

Data:

0-3:		max 1001	10er Year 	up to 90	if higher 1001 /SpecialData1 mark

4-7:		max 1001	1er Year 	up to 9		if higher 1001 /Special Use mark

8:		max 1		10er Month 	up to 10

9-12:		max 1001	1er Month 	up to 9

13+14:		max 11	10er Day 	up to 30

15-18:		max 1001	1er Day 	up to 9

19+20:		max 10	10er Hours 	up to 20

21-24:		max 1001	1er Hours 	up to 9

25-27:		max 101	10er Minutes 	up to 50

28-31:		max 1001	1er Minutes 	up to 9

32-34:		max 101	10er Minutes 	up to 50

35-38:		max 1001	1er Minutes 	up to 9

Temperature:

0-7:		max 11111111	*0,5° Celsius

ShortCode Info:

If Short Code is “1” 2MSB Value like Signed have to be Deleted / Added

Writing / Reading

Special Data:

InfoBit:	001

0-3:		1010		DataEnd

1011		MemoryEnd

1100		1Times before Measured Temperature

1101		2Times before Measured Temperature

1110		3Times before Measured Temperature

1111		4Times before Measured Temperature

InfoBit:	010

0-3:		max. 1111	Just Measured Temperature again

4-7:		1010		1Times + ( “Bit0-3” * 6 ) only takes Place if Bit 0-3 higher 0000

1011		2Times + ( “Bit0-3” * 6 )

1100		3Times + ( “Bit0-3” * 6 )

1101		4Times + ( “Bit0-3” * 6 )

1110		5Times + ( “Bit0-3” * 6 )

1111		6Times + ( “Bit0-3” * 6 )

Refresh rate:

- 1second

- 5seconds

- 1Minute

- 5Minutes (experimental)

Structure:

Data begins at 0x0000 in Memory, followed by Data1 Code. With every Measuring a Data2 Code will be added. If Measuring will be paused or stopped a new Data1 is written.

After every writing of Data a SpecialData1 “DataEnd” which is overwritten by the next Data will be written.

If End of Memory is reached a SpecialData1 “MemoryEnd” will be written and the monitoring Stopped.

To Overwrite Data, the Memory can be overwritten without Formatting.

The Memory can be cleard by writing SpecialData1 “DataEnd” at the position of 0x0000.

Every DataCode is to be understood with the InfoBits in FRONT of them.

Example showing 34 measurements:

001 0001 0001 0 0111 10 0001 01 0000 010 0011 010 0101 110 100010 010 0101 1101 001 1010