INTSER inserts itself into memory below command.com (it uses

the "terminate but stay resident" DOS call), and changes the ROM

bios interupt vectors for RS232 I/O to use interupt driven I/O.

Thus if you have a program that uses the ROM BIOS to do serial I/O,

and the program has been limited in speed by the poor performance

of the standard ROM routines, all you need to do is run INTSER first,

and the program will suddenly magically work at speeds up to 19200 bps

if you use the initialisation for 110 bps. (i.e. MODE COM1;110,N,8,1 )

This is also useful for writing programs that will be transportable

to IBM lookilikes that emulate the ROM BIOS, but not the same hardware

configuration.

Instructions for use:

edit INTSER to reflect your choice of buffer size and

handshaking characters.

assemble using the macro assembler.

link using LINK.

convert to COM file using EXE2BIN.

delete .EXE file.

INTSER works with DOS 1.1 and higher.

Limitations:

Various programs will break INTSER by virtue of

not leaving the serial port as it was found or

reprogram the Interruptcontroller.  The most

notable amoung these are  BASIC.  If this happens,

it will probably be necessary to power cycle the

system.

INTSER is set up for COM1 only.  It could easilly

be converted to use COM2, and with slightly more

difficulty, both.

INTSER only receives characters on an interrupt basis,

characters output to the serial port are sent in the

old slow manner.  Thus INTSER is not particularly

useful for, say, a serial printer spooler.

INTSER does not check the various other RS232 signals

that the orignal ROM routine checks.  Thus, depending on

exactly how the hardware works, hardware handshaking

may not work  or any timeout may occur.

870828   wb