DESIGN DATA FOR INTERDIGITAL BANDPASS FILTERS

Rook & Taylor (1970); Hinshaw N6JH & Monemzadeh (Ham Radio, Jan 1985)

I/O revision by Ian White G3SEK 1989-91

Press M for millimetres,  or I for inches  > mm

Groundplane spacing, top-bottom (mm)  : 12

Rod diameter (mm)  : 5

Overhang from centre of first & last rod [ 8.40mm]  : 8.4

Number of elements (7 or less) [ 3]  : 5

Passband ripple (dB, 0=Butterworth)    [0.00]  : 0

Centre frequency (GHz)   [ 1.000]  : 1.7

Bandwidth (MHz)  [ 85.0]  : 30

Input/output impedance  [50Ohm]  : 50

Number of plotting points required at each side of centre, and freq step -

Points  [10]  : 25

Step (MHz)  [ 15]  : 4

DESIGN DATA FOR  5  POLE INTERDIGITAL FILTER

Butterworth response

Centre freq.          1.7000 GHz

Cutoff freqs          1.6850 and  1.7150 GHz

Ripple bandwidth      0.00000 GHz

3dB bandwidth      0.03000 GHz

Fractional bandwidth  0.0176                                      GHz    dB

Filter  Q         57                                           > 1.600  82.39

Estimated Qu    1355                                           > 1.604  80.62

Loss based on this Qu    1.2 dB                                > 1.608  78.77

Delay at band centre    34.3 ns                                > 1.612  76.84

> 1.616  74.82

> 1.620  72.70

> 1.624  70.47

> 1.628  68.12

> 1.632  65.64

FREQUENCY REJECTION INFORMATION                               * 1.636  63.01

*    1.640  60.21

*       1.644  57.21

*           1.648  53.99

*              1.652  50.52

*                  1.656  46.74

*                      1.660  42.60

*                          1.664  38.02

*                                1.668  32.91

*                                     1.672  27.12

*                                            1.676  20.45

*                                                    1.680  12.73

*                                                            1.684   4.63

*                                                                1.688   0.44

*                                                                1.692   0.01

*                                                                1.696   0.00

*                                                                1.700   0.00

*                                                                1.704   0.00

*                                                                1.708   0.01

*                                                                1.712   0.44

*                                                            1.716   4.63

*                                                    1.720  12.73

*                                            1.724  20.45

*                                     1.728  27.12

*                                1.732  32.91

*                          1.736  38.02

*                      1.740  42.60

*                  1.744  46.74

*              1.748  50.52

*           1.752  53.99

*       1.756  57.21

*    1.760  60.21

* 1.764  63.01

> 1.768  65.64

> 1.772  68.12

> 1.776  70.47

> 1.780  72.70

> 1.784  74.82

> 1.788  76.84

> 1.792  78.77

> 1.796  80.62

> 1.800  82.39

Width (= quarter-wavelength)    44.09mm

Height                          12.00mm

Rod diameter                     5.00mm

External 50 Ohm lines connect at  3.68mm from ground end

End covers are required unless overhang is larger than default value

Ele #      Length                               Centre-to-centre    Cumulative

(edge)                      ┌─ │ ──────────┐                           0.00mm

│  │           │         8.40mm

1         40.51mm         ▐█████████████ │                           8.40mm

│              │        18.41mm

2         40.37mm         │ █████████████▌                          26.81mm

│              │        20.69mm

3         40.37mm         ▐█████████████ │                          47.50mm

│              │        20.69mm

4         40.37mm         │ █████████████▌                          68.20mm

│              │        18.41mm

5         40.51mm         ▐█████████████ │                          86.61mm

│  │           │         8.40mm

(edge)                      └─ │ ──────────┘                          95.01mm