# Dallas 1-Wire(R) Master Interrupt-driven File Documentation

## main.c File Reference

### Detailed Description

Atmel Corporation

- File: main.c

- Compiler: IAR EWAAVR 3.20a

- Support mail: avr@atmel.com

- Supported devices: All AVRs with UART or USART module.

- Application Note: AVR318 - Dallas 1-Wire(R) master.

- Description: Example on how to use the interrupt-driven UART implementation of the 1-Wire(R) protocol.

Revision

1.7

Date

Thursday, August 19, 2004 14:27:16 UTC

### Functions

- void main (void)

Code example for the interrupt-driven driver.

- void OWI_StateMachine ()

The state machine that controls communication on the 1-Wire bus.

### Variables

- unsigned char * OWIDataBuffer

- OWIflags OWIStatus

- signed int temperature

### Function Documentation

#### void main (void)

This code example assumes that there is a DS1820 temperature sensor connected to the 1-Wire bus. The temperature sensor is continously polled, and the temperature reading is output to PORTB.

#### void OWI_StateMachine ()

This function is called from main every time the 1-Wire driver is not busy. The state machine will read the temperature from a DS1820 temperature sensor, crc check it, and put it in the global variable temperature if everything is OK.

## OWIInterruptDriven.h File Reference

### Detailed Description

Atmel Corporation

- File: OWIInterruptDriven.h

- Compiler: IAR EWAAVR 3.20a

- Support mail: avr@atmel.com

- Supported devices: All AVRs with UART or USART module.

- Application Note: AVR318 - Dallas 1-Wire(R) master.

- Description: Defines used in the interrupt-driven 1-Wire(R) driver.

Revision

1.7

Date

Thursday, August 19, 2004 14:27:16 UTC

## OWIIntFunctions.c File Reference

### Detailed Description

Atmel Corporation

- File: OWIIntFunctions.c

- Compiler: IAR EWAAVR 3.20a

- Support mail: avr@atmel.com

- Supported devices: All AVRs with UART or USART module.

- Application Note: AVR318 - Dallas 1-Wire(R) master.

- Description: Functions and Interrupt Service Routines that handles automated transmission and reception of 0-255 bits.

Revision

1.7

Date

Thursday, August 19, 2004 14:27:18 UTC

### Functions

- void OWI_DetectPresence (void)

Send the Reset signal on 1-Wire bus to receive presence signal from slave devices.

- void OWI_Init (void)

Initialization of the U(S)ART for 1-Wire communication.

- void OWI_ReceiveData (unsigned char *data, unsigned char dataLength)

Receive data from the 1-Wire bus.

- void OWI_TransmitData (unsigned char *data, unsigned char dataLength)

Transmit data on the 1-Wire bus.

- __interrupt void UART_RXC_ISR ()

UART Receive Complete Interrupt Service Routine.

- __interrupt void UART_UDRE_ISR ()

UART Data Register Empty Interrupt Service Routine.

### Variables

- unsigned char OWIBufferLength

- unsigned char * OWIDataBuffer

- OWIflags OWIStatus

### Function Documentation

#### void OWI_DetectPresence (void)

This function configures the UART module for transmission of the Reset signal. The busy flag is TRUE until the 1-Wire driver has completed the Reset/Presence sequence. when the busy flag becomes FALSE, the presence flag indicates whether a Presence signal was received.

#### void OWI_Init (void)

This function configures the U(S)ART for 1-Wire communication.

Note:

Parameters are set in "OWIInterruptDriven.h"

#### void OWI_ReceiveData (unsigned char * data, unsigned char dataLength)

This function configures the UART module for reception of data. The busy flag is TRUE until reception is complete.

Parameters:

data A pointer to the location in memory where the received data should be placed

dataLength The number of bits to receive.

#### void OWI_TransmitData (unsigned char * data, unsigned char dataLength)

This function configures the UART module for transmission of data. The busy flag is TRUE until transmission is complete.

Parameters:

data Pointer to the data to be transmitted.

dataLength The number of bits to be transmitted.

#### __interrupt void UART_RXC_ISR ()

This ISR takes care of reception and interpretation of signals from the 1-Wire bus.

#### __interrupt void UART_UDRE_ISR ()

This ISR is run every time the UART transmit buffer is empty, thus automating the process of transmitting and receiving bits.

## OWIIntFunctions.h File Reference

### Detailed Description

Atmel Corporation

- File: OWIIntFunctions.h

- Compiler: IAR EWAAVR 3.20a

- Support mail: avr@atmel.com

- Supported devices: All AVRs with UART or USART module.

- Application Note: AVR318 - Dallas 1-Wire(R) master.

- Description: Header file for OWIIntFunctions.c

Revision

1.7

Date

Thursday, August 19, 2004 14:27:18 UTC

### Data Structures

- union OWIflags

Union that holds 1-Wire flags.

### Functions

- void OWI_DetectPresence (void)

Send the Reset signal on 1-Wire bus to receive presence signal from slave devices.

- void OWI_Init (void)

Initialization of the U(S)ART for 1-Wire communication.

- void OWI_ReceiveData (unsigned char *data, unsigned char dataLength)

Receive data from the 1-Wire bus.

- void OWI_TransmitData (unsigned char *data, unsigned char dataLength)

Transmit data on the 1-Wire bus.

### Function Documentation

#### void OWI_DetectPresence (void)

This function configures the UART module for transmission of the Reset signal. The busy flag is TRUE until the 1-Wire driver has completed the Reset/Presence sequence. when the busy flag becomes FALSE, the presence flag indicates whether a Presence signal was received.

#### void OWI_Init (void)

This function configures the U(S)ART for 1-Wire communication.

Note:

Parameters are set in "OWIInterruptDriven.h"

#### void OWI_ReceiveData (unsigned char * data, unsigned char dataLength)

This function configures the UART module for reception of data. The busy flag is TRUE until reception is complete.

Parameters:

data A pointer to the location in memory where the received data should be placed

dataLength The number of bits to receive.

#### void OWI_TransmitData (unsigned char * data, unsigned char dataLength)

This function configures the UART module for transmission of data. The busy flag is TRUE until transmission is complete.

Parameters:

data Pointer to the data to be transmitted.

dataLength The number of bits to be transmitted.