// Include Standard LIB  files

#include "Board.h"

#include <stdio.h>

#define RTTC_INTERRUPT_LEVEL		0

#define PIV_200_MS 		600000		//* 200 ms for 48 MHz

// Global Variable

char message[80];

unsigned int value;

//*----------------------------------------------------------------------------

//* Function Name       : Periodic_Interval_Timer_handler

//* Object              : C handler interrupt function called by the interrupts

//*                       assembling routine

//*----------------------------------------------------------------------------

void Periodic_Interval_Timer_handler(void)

{

unsigned int status;

//* Interrupt Acknowledge

status = AT91C_BASE_PITC->PITC_PIVR;

status =status;

AT91C_BASE_DBGU->DBGU_THR = '*';

//* Read the output state

if ((AT91F_PIO_GetInput(AT91C_BASE_PIOA) & LED1 ) == LED1 )   AT91F_PIO_ClearOutput( AT91C_BASE_PIOA, LED1 );

else  AT91F_PIO_SetOutput( AT91C_BASE_PIOA, LED1 );

}

//*----------------------------------------------------------------------------

//* Function Name       : FIQ_handler

//* Object              : Irq Handler called by the FIQ interrupt with AT91

//*                       compatibility

///*----------------------------------------------------------------------------

void FIQ_handler(void)

{

value = AT91C_BASE_PITC->PITC_PIIR;

//* Read the output state

if ( (AT91F_PIO_GetInput(AT91C_BASE_PIOA) & LED3 ) == LED3 )

{

AT91F_PIO_ClearOutput( AT91C_BASE_PIOA, LED3 );

}

else

{

AT91F_PIO_SetOutput( AT91C_BASE_PIOA, LED3 );

}

//* while SW3 is push wait

while ( (AT91F_PIO_GetInput(AT91C_BASE_PIOA) & SW3_MASK ) != SW3_MASK );

}

//*----------------------------------------------------------------------------

//* Function Name       : main

//* Object              : Main interrupt function

//*----------------------------------------------------------------------------

int main( void )

//* Begin

{

char val;

// First, enable the clock of the PIOB

AT91F_PMC_EnablePeriphClock ( AT91C_BASE_PMC, 1 << AT91C_ID_PIOA ) ;

//* then, we configure the PIO Lines corresponding to LED1 to LED8

//* to be outputs. No need to set these pins to be driven by the PIO because it is GPIO pins only.

AT91F_PIO_CfgOutput( AT91C_BASE_PIOA, LED_MASK ) ;

//* Clear the LED's. On the EB55 we must apply a "1" to turn off LEDs

AT91F_PIO_SetOutput( AT91C_BASE_PIOA, LED_MASK ) ;

//* Init Real Time Timer

//* Open Timer 0 interrupt

AT91F_AIC_ConfigureIt ( AT91C_BASE_AIC, AT91C_ID_SYS, RTTC_INTERRUPT_LEVEL,AT91C_AIC_SRCTYPE_INT_POSITIVE_EDGE, Periodic_Interval_Timer_handler);

//* Set the interrupt mo

AT91C_BASE_PITC->PITC_PIMR = AT91C_PITC_PITEN | AT91C_PITC_PITIEN | PIV_200_MS;  //  IRQ enable CPC

//*  RTTC_RTAR

AT91F_AIC_EnableIt (AT91C_BASE_AIC, AT91C_ID_SYS);

//* open  FIQ interrupt

AT91F_PIO_CfgPeriph(AT91C_BASE_PIOA,SW1_MASK,0);

AT91F_AIC_ConfigureIt (AT91C_BASE_AIC, AT91C_ID_FIQ, 0,AT91C_AIC_SRCTYPE_EXT_NEGATIVE_EDGE, FIQ_handler);

AT91F_AIC_EnableIt (AT91C_BASE_AIC, AT91C_ID_FIQ);

//* generate FIQ interrupt by software

AT91F_AIC_Trig (AT91C_BASE_AIC,AT91C_ID_FIQ) ;

//* open external PIO interrupt

//* define switch SW3 at PIO input for interrupt IRQ loop

AT91F_PIO_CfgInput(AT91C_BASE_PIOA, SW3_MASK );

//* Infinit loop

for (;;)

{

//* Interrupt Acknowledge

if (AT91F_US_Get(&val))

{

value = AT91C_BASE_PITC->PITC_PIIR;

}

}

//* End

}