# Scope

This document describes how to test USB Device CDC example.

# Preparation

Host

Personal computer running Windows Xp or Windows 7.

Device

A board, i.e. twrk70f120m, which is running dev_cdc_virtual_com example.

# Steps

Follow the steps to run the CDC Virtual Com demo.

- Plug-in the CDC device which is running dev_cdc_virtual_com  example into PC. You will see a COM port enumerated in Device Manager. If it prompts for CDC driver installation. Refer to the next section to see how to install CDC driver.

- Open the COM port in a terminal tool, i.e. Tera Term.

- Type some characters and you can see them echoed back from the COM port.

Note: Since there is no dynamic detection between host and device, the COM port must be closed from the terminal tool prior to plug-out the CDC device. Or the CDC device won’t get recognized next time you plug-in with the COM port still opened.

# How to install CDC driver for virtual_com and msd_cdc composite example

### Below are the steps to install CDC driver on Windows 7, while on Windows XP the similar way apply.

## Step 1. Click “Update Driver Software…”

## Step 2. Choose “Browse…”

## Step 3. Select “Let me pick…”

## Step 4. Navigate to your CDC driver location.

## Step 5. Press “Next”.

## Step 6. Ignore the warning and press “Yes”.

## Step 7. Now the CDC driver should has been installed successfully.