FUNCTION TABLE

See note 1.

Note

H = HIGH voltage level;

L = LOW voltage level;

L>H = LOW-to-HIGH transition;

H>L = HIGH-to-LOW transition;

Z = high-impedance OFF-state;

n.c. = no change;

X = don’t care.

INPUT 					OUTPUT	FUNCTION

SH_CP	ST_CP	OE	MR	DS	Q7’	Qn

X	X	L	L	X	L	n.c.	a LOW level on MR only affects the shift registers

X	L>H	L	L	X	L	L 	empty shift register loaded into storage register

X	X	H	L	X	L	Z	shift register clear; parallel outputs in high-impedance

OFF-state

L>H	X	L	H	H	Q6’	n.c.	logic high level shifted into shift register stage 0;

contents of all shift register stages shifted through, e.g.

previous state of stage 6 (internal Q6’) appears on the

serial output (Q7’)

X	L>H	L	H	X	n.c.	Qn’	contents of shift register stages (internal Qn’) are

transferred to the storage register and parallel output

stages

L>H	L>H	L	H	X	Q6’	Qn’	contents of shift register shifted through; previous

contents of the shift register is transferred to the

storage register and the parallel output stages