This file documents the changes made to Intel's MCS-51(tm) BASIC V1.1 or

better known as the BASIC in the 8052AH-BASIC chip. These changes were

made to allow the BASIC to run on an 8031/8051 uP system in external EPROM.

Dan Karmann

(303)538-4075

Files:

BASIC31.DOC - This document file.

BASIC31.HEX - Intel Hex formated file of modified BASIC-52.

Summary:

All 8052-specific functions/statements have been disabled.

Due to the fact that the EPROM programming features do not work

when executing BASIC-52 from external EPROM, all EPROM programming

features have been disabled.

Timer 1 defaults to the baud rate generation for the console port

except during execution of any commands that send output to the

list port and during the execution of the PWM statement.

All other features/functions are intact.

Changes:

Timer 2 Overflow Interrupt disabled.

- replaced with RETI instruction.

TIMER2 special function operator disabled.

- generates ERROR: BAD SYNTAX message.

T2CON special function operator disabled.

- generates ERROR: BAD SYNTAX message.

RCAP2 special function operator disabled.

- generates ERROR: BAD SYNTAX message.

PROG command disabled.

- generates ERROR: PROGRAMMING message.

FPROG command disabled.

- generates ERROR: PROGRAMMING message.

PROGx command disabled.

- generates ERROR: PROGRAMMING message.

FPROGx command disabled.

- generates ERROR: PROGRAMMING message.

PGM statement disabled.

- generates ERROR: PROGRAMMING message.

Timer 1 used for serial console port. (19200 baud is maximum)

EPROM baud rate value at 8000H converted to Timer 1 value when

used; i.e. - value stored in EPROM by BASIC-52 still works.

Autobaud rate value converted to Timer 1 value.

Reserved location 44H in internal memory used for Timer 1 baud

rate value.

Timesharing of Timer 1 support added to PH0.#, PH1.#, PRINT#,

P.#, ?#, and LIST# statements so that when sending output to

the list device, Timer 1 is used for the list device baud rate.

Timer 1 is restored to the baud rate timer for the console port

when output to the list device is finished.

Internal stack overflow checking modified to recognize that the

8031/8051 has only 128 bytes of internal RAM instead of 256 bytes.

Timesharing of Timer 1 support added to the PWM statement so that

when generating output on the PWM pin, Timer 1 is used for that

timing. Timer 1 is restored to the baud rate timer for the console

port when the PWM output is finished.

Sign-on version number changed to V1.2.

"Modified by ..." text added to ^W^C message.

Enhancements:

Backspace support added to line editing - in addition to the

Delete character.

The floating point subtraction routine modifications and the

ASCII-TO-BINARY performance improvements from the OCTOBER 1991

issue of Elektor Electronics USA have been added. (Note: The

multiplication routine modifications from the NOVEMBER 1991 issue

of Elektor Electronics USA were not included due to the fixes not

performing as indicated in the article)

Anomalies:

Due to the timesharing of Timer 1 during sending output to the

list device (PRINT#, LIST#, etc.), a ^C (control C) will not stop

the list output because Timer 1 is used for the output baud rate

and the console serial port is deactivated.

Due to the smaller internal RAM in the 8031/8051, the INTERNAL STACK

is defaulted to 51 bytes instead of the 179 bytes in the 8052. This

may cause some programs to generated an "ERROR: I-STACK". This is

typically caused by too many nested levels of parenthesis "()", and

can usually be made to work by breaking the expression into several

less complex expressions.

Notes:

These code changes have been tested, but is not guaranteed to

be bug free (what software is?!).

For those who haven't discovered this, BASIC-52 has a keystroke

sequence ^W^C (control-W followed by a control C) that will

display the developers' name, company, and date.