Listings for 09di2198

Back.cpp

//background program

//developed with Turbo C++

//operating under DOS

#include "mem.h"

#include <conio.h>

#define RTS 0x2

//prototypes

void setupUART (void);

void deAssert (int ControlPin);

void Assert (int ControlPin);

unsigned char SerialIn(void);

void UARTrx(void);

void UARTtx(void);

unsigned char UART_TX_clear( void);

unsigned char SerialOut (void);

unsigned int checksum (unsigned char NumberOfBytes);

void main (void)

{

unsigned int j;

comport=1;

//setting to COM1

module_address=0xa;

//PC address=10 decimal

//other transmission constants

setupUART();

//initialise the UART

capture_enabled=0;

rx_pnt=0;

//initialise variables

Assert(RTS);

//turn the RS485 buffer to receive

while (1)

{

/*the actions are divided into several states as indicated by the variable "phase".

Phase=0- waiting for a complete serial message

Phase=1- preparing a response

Phase=2-wait for end of transmission

Phase=3- wait for message to completely clear the UART (buffers empty) & then

re-enable reception (turn RS485 buffer around)*/

switch (phase)

{

case 0:

if (SerialIn())

//checking for complete message received

{

//now to process the input

phase++;

UARTtx();

//prepare UART to send

}

break;

case 1:

//prepare to transmit

rx_buff[0]=0x0;     //destination address

rx_buff[1]=0x13;    //response

rx_buff[2]=0xff;

//set last byte.

number_of_characters=3;

//variable for transmit routine

rx_pnt=0;

//intitialise the fetch pointer

phase++;

break;

case 2:

if (SerialOut())

{//at the end of the message

//bump on to wait for complete transmission

phase++;

}

break;

case 3:

//wait for message to clear

if (UART_TX_clear())

{

UARTrx();

phase=0;

}

break;

default:

break;

}

if (kbhit())

{//terminate execution if any key pressed.

break;

}

}

}

mem.cpp

unsigned int SerialPortBase;

//base address of the UART

unsigned int comport;

//setting for com1-4

unsigned char capture_enabled;

//flag that a capture is in process

//-public because the SerialIn routine would lose the info on exit

unsigned int rx_pnt;

//pointer to the serial buffer

int rx_buff[40];

//serial buffer

//rx_ is a misnomer, the buffer is used for reception and transmission

unsigned int module_address;

//the address of the PC on the network

unsigned int phase;

//current state of operations

int number_of_characters;

//number of characters to be transmitted

mem.h

extern unsigned int SerialPortBase;

//base address of the UART

extern unsigned int comport;

//setting for com1-4

extern unsigned char capture_enabled;

//flag that a capture is in process

//-public because the SerialIn routine would lose the info on exit

extern unsigned int rx_pnt;

//pointer to the serial buffer

extern int rx_buff[40];

//serial buffer

//rx_ is a misnomer, the buffer is used for reception and transmission

extern unsigned int module_address;

//the address of the PC on the network

extern unsigned int phase;

//current state of operations

extern int number_of_characters;

//number of characters to be transmitted

serial.cpp

/*serial communications- no interrupt facility

*/

#include <dos.h>

#include "mem.h"

#define TXR 0  //transmit register (offset 0)

#define RXR 0  //receive register (offset 0)

#define IER 1       //Interrupt Enable register (offset 1)

#define IIR 2       //Interrupt Identification register (offset 2)

#define LCR 3  //Line Control Register

#define DLAB 0x8              //Divisor Latch Bit

#define EVEN_PARITY 0x18 //Enable parity and Even parity

#define STICK 0x28            //Enable stick parity (and bit 9 =1)

#define TB9_0 0x10            //transmit a 0 in bit 9

#define MCR 4  //Modem Control Register

#define RTS 0x2               //RTS line

#define LSR 5       //Line Status Register

#define DR 0x1           //Data Ready

#define OE 0x2           //Overrun error

#define PE 0x4           //Parity error

#define FE 0x8           //Framing error

#define THRE 0x20             //Holding buffer empty

#define TSRE 0x40             //Transmit buffer empty

#define DLL 0       //Divisor Latch (LSB)

#define DLH 1  //Divisor Latch (MSB)

void Assert(int ControlLine)

{//USE TO SET CONTROL BITS IN MCR (Modem Control Register)

int presentValue;

presentValue = inp(SerialPortBase + MCR);

//fetch the current settings of the Modem Control Register

outp(SerialPortBase + MCR,(presentValue | ControlLine));

//force the associated control line active

}

void deAssert (int ControlLine)

{//USE TO TURN OFF CONTROL BITS IN MCR (Modem Control Register)

int presentValue;

//fetch the current settings of the Modem Control Register

presentValue = inp(SerialPortBase + MCR);

outp(SerialPortBase + MCR,(presentValue & ~(ControlLine)));

//force the associated control line inactive

}

void setupUART (void)

{//serial port setup

long baud;

unsigned char presentValue;

int divisor;

int setting;

baud=10416;

//the 8250/16450/16550 can handle almost any baud rate

switch (comport)

//set up the I/O port address based on the desired COM channel

{

case 1: //COM1:

SerialPortBase = 0x3f8;

break;

case 2: //COM2:

SerialPortBase =0x2f8;

break;

case 3: //COM3:

SerialPortBase =0x3e8;

break;

case 4: //COM4:

SerialPortBase =0x2e8;

break;

}

divisor=(int)(115200L/baud);

//calculate the divisor for the desired baud rate

presentValue = inp(SerialPortBase + LCR);

//fetch the current value in order to OR it with DLAB

outp(SerialPortBase + LCR,(presentValue | DLAB));

//and now for the divisors

outp(SerialPortBase + DLL,(divisor & 0x00FF));

outp(SerialPortBase + DLH,((divisor >> 8) & 0x00FF));

//remove DLAB

outp(SerialPortBase + LCR,presentValue);

//now for the other bits

setting = 0x3; //8 data bits, 1 stop bit

setting |= EVEN_PARITY; //even parity

outp(SerialPortBase + LCR,setting);

//ensure interrupt is off

outp(SerialPortBase + IER,0);

//and fifo is off

outp(SerialPortBase+IIR,0);

}

unsigned char even_parity (int check_byte)

{

//checks the least significant 8 bits

//if even number of ones then parity is even

//returns 0 for even number of ones

//returns 1 for odd number of ones

unsigned char i,j;

j=0;

for (i=0;i<8;i++)

{

if (check_byte & 0x1)

j++;

check_byte=check_byte>>1;

}

return (j & 0x1);

//odd or even is reflected in the LSB

}

unsigned char SerialIn (void)

{

//receives and buffers incoming data until the termination byte is received (0xff with TB9=1)

//returns with a "1" value if the message is complete.

unsigned int lsr,received_byte;

unsigned char bit9;

unsigned char message_complete;

unsigned char dummy;

message_complete=0;

lsr=inp(SerialPortBase+LSR);

if ((lsr & DR) != 0)

{

received_byte = inp(SerialPortBase + RXR);

rx_buff[rx_pnt]=received_byte;

//check for errors

if ((lsr & (OE | FE))!=0)

{//if an error is detected then ..

rx_pnt=0;

//re-initiate data capture

}

else {

//no framing or parity errors

if (even_parity(received_byte))

{

//here there is odd parity for the received byte

//if no error was detected then bit 9=1

if (lsr & PE)

{

bit9=0;

}

else {

bit9=1;

}

}

else {

//even parity for the received byte

//if no parity error detected, bit 9=0

if (lsr & PE)

{//error detected

bit9=1;

}

else {

bit9=0;

}

}

if (bit9)

{

if (received_byte ==0xff)

{//0xff with bit 9=1 is EOT

if (capture_enabled)

{//had this reception been enabled

// by a previous \valid address

message_complete=1;

//flag end of message

}

else {

rx_pnt=0;

//ensure pointer is ready for next address

}

}

else {

if (received_byte==module_address)

{

//indicate that capture is enabled

capture_enabled=1;

rx_pnt++;

}

else {//address, but not for this address

//so restart.

capture_enabled=0;

rx_pnt=0;

}

}

}

else {// bit 9 is 0 so a data bit

if (capture_enabled)

{

rx_pnt++;

}

}

}

}

return (message_complete);

}

void UARTrx (void)

{

//prepares the UART to receive. i.e. with parity

int setting;

Assert (RTS);

//turn the RS485 buffer to receive

setting = 0x3;

setting |= EVEN_PARITY;

outp(SerialPortBase + LCR,setting);

}

void UARTtx (void)

{

//prepares UART to transmit with STICK bit

int setting;

deAssert(RTS);

//turn the RS485 buffer to transmit

setting = 0x3;

setting |= STICK;

//TB9_0, sets 1 in bit, must clear for 1

outp(SerialPortBase + LCR,setting);

}

unsigned char SerialOut (void)

{//send a character onto the UART send register

//if first byte of last byte then 9th bit =1, else =0;

//routine returns non-zero if last byte sent.

int setting;

unsigned char k;

k=0; //flag that process is incomplete

setting = 0x3;

setting |=STICK;

if ((rx_pnt==0)|| (rx_pnt==(number_of_characters-1)))

{

//set 9th bit to 1 for first or last byte

//(automatically done in STICK constant)

;

}

else {

//set 9th bit to 0

setting |=TB9_0;

}

outp(SerialPortBase + LCR,setting);

if ((inp(SerialPortBase + LSR) &  (THRE | TSRE)) == (THRE | TSRE))

{

//wait for transmit registers to go empty

outp(SerialPortBase + TXR,rx_buff[rx_pnt]);

rx_pnt++;

if (rx_pnt>=number_of_characters)

k=1;

}

return k;

}

unsigned char UART_TX_clear( void)

{

//returns non-zero if there is no data left in either transmit buffer of the UART

//returns 0 if still waiting for characters to leave the UART

unsigned char k;

k=0;

if ((inp(SerialPortBase + LSR) &  (THRE | TSRE)) == (THRE | TSRE))

k=1;

return k;

}