$regfile = "8515def.dat"

$crystal = 4000000

Dim Ctr As Byte

Dim Dataword As Word

Dim Command As Byte

Dim Dis As String * 20

Dim Calc As Single

Dim Calc2 As Single

Dim Rhlinear As Single

Dim Rhlintemp As Single

Dim Tempc As Single

Dim Tempf As Single

Const C1 = -4

Const C2 = 0.0405

Const C3 = -0.0000028

Const T1c = .01

Const T2 = .00008

Const T1f = .018

Sck Alias Portb.0

Dataout Alias Portb.1

Datain Alias Pinb.1

Redled Alias Portb.2

Declare Sub Getit()

Ddrb = &B11111111

Config Pinb.0 = Output

Config Pinb.1 = Output

Set Dataout

For Ctr = 1 To 12

Set Sck

Waitus 2

Reset Sck

Waitus 2

Next Ctr

Do

Command = &B00000011

Call Getit

Tempc = T1c * Dataword

Tempc = Tempc - 40

Dis = Fusing(tempc , "###.##")

Print "Temperatur = " ; Dis ; " (C)"

Command = &B00000101

Call Getit

Calc = C2 * Dataword

Calc2 = Dataword * Dataword

Calc2 = C3 * Calc2

Calc = Calc + C1

Rhlinear = Calc + Calc2

Calc = T2 * Dataword

Calc = Calc + T1c

Calc2 = Tempc - 25

Calc = Calc2 * Calc

Rhlintemp = Calc + Rhlinear

Dis = Fusing(rhlintemp , "##.##")

Print "rel. Feuchte " ; Dis

Print

Wait 1

Loop

Sub Getit()

Local Datavalue As Word

Local Databyte As Byte

Set Sck

Reset Dataout

Reset Sck

Set Sck

Set Dataout

Reset Sck

Shiftout Dataout , Sck , Command , 1

Ddrb = &B11111101

Config Pinb.1 = Input

Set Sck

Reset Sck

Waitus 10

Bitwait Pinb.1 , Reset

Shiftin Datain , Sck , Databyte , 1

Datavalue = Databyte

Ddrb = &B11111111

Config Pinb.1 = Output

Reset Dataout

Set Sck

Reset Sck

Ddrb = &B11111101

Config Pinb.1 = Input

Shiftin Datain , Sck , Databyte , 1

Shift Datavalue , Left , 8

Datavalue = Datavalue Or Databyte

Dataword = Datavalue

Ddrb = &B11111111

Config Pinb.1 = Output

Reset Dataout

Set Sck

Reset Sck

Ddrb = &B11111101

Config Pinb.1 = Input

Shiftin Datain , Sck , Databyte , 1

Ddrb = &B11111111

Config Pinb.1 = Output

Set Dataout

Set Sck

Reset Sck

End Sub

End