Capture / Compare Unit 2 (CAPCOM2)

Macros:

Macro:

CC2_vStartTmr(TimerNr)

Description:

This macro starts the selected CAPCOM2 timer. The timer

continues counting from where it stopped.

The following definitions for TimerNr are available:

CC2_TIMER_7

CC2_TIMER_8

(see @Defines in header file)

Returnvalue:

None

Parameters:

TimerNr:

Name of the CAPCOM2 timer to be started

Macro:

CC2_vStopTmr(TimerNr)

Description:

This macro stops the selected CAPCOM2 timer. The content of

the timer register remains unchanged.

The following definitions for TimerNr are available:

CC2_TIMER_7

CC2_TIMER_8

(see @Defines in header file)

Returnvalue:

None

Parameters:

TimerNr:

Name of the CAPCOM2 timer to be stopped

Macro:

CC2_vClearTmr(TimerNr)

Description:

This macro stops the selected CAPCOM2 timer and sets the

timer register to 0.

The following definitions for TimerNr are available:

CC2_TIMER_7

CC2_TIMER_8

(see @Defines in header file)

Returnvalue:

None

Parameters:

TimerNr:

Name of the CAPCOM2 timer to be cleared

Macro:

CC2_uwReadTmr(TimerNr)

Description:

This macro returns the content of the CAPCOM2 timer. The

timer is not stopped. CPU access to the timer register has

a higher priority than a timer increment or a reload.

The following definitions for TimerNr are available:

CC2_TIMER_7

CC2_TIMER_8

(see @Defines in header file)

Returnvalue:

Current 16-bit value of the timer register

Parameters:

TimerNr:

Name of the CAPCOM2 timer to be readed

Macro:

CC2_vLoadTmr(TimerNr, Value)

Description:

This macro loads the timer register of the selected CAPCOM2

timer with the requested value. The timer need not be

stopped for this happen as a CPU access to the timer

register has a higher priority than a timer increment or a

reload.

The following definitions for TimerNr are available:

CC2_TIMER_7

CC2_TIMER_8

(see @Defines in header file)

Returnvalue:

None

Parameters:

TimerNr:

Name of the CAPCOM2 timer to be loaded

Value:

16-bit Value with which the CAPCOM2 timer is to be loaded

Macro:

CC2_vSetReload(TimerNr, Value)

Description:

This macro loads the reload register of the selected

CAPCOM2 timer.

The following definitions for TimerNr are available:

CC2_TIMER_7

CC2_TIMER_8

(see @Defines in header file)

Returnvalue:

None

Parameters:

TimerNr:

Name of the CAPCOM2 timer to be used

Value:

16-bit Value with which the CAPCOM2 timer reload register

is to be loaded

Macro:

CC2_vSetCCxReg(RegName, Value)

Description:

This macro loads the selected CAPCOM2 register with the

parameter Value.

The following definitions are available for RegName (only

for enabled CAPCOM2 channels):

CC2_CC_16 .. CC2_CC_31

(see @Defines in header file)

Returnvalue:

None

Parameters:

RegName:

Name of the CAPCOM2 channel to be loaded

Value:

16-bit Value with which the CAPCOM2 register is to be loaded

Macro:

CC2_uwGetCCxReg(RegName)

Description:

This macro returns the content of the selected CAPCOM2

register.

The following definitions are available for RegName (only

for enabled CAPCOM2 channels):

CC2_CC_16 .. CC2_CC_31

(see @Defines in header file)

Returnvalue:

Current 16-bit value of the CAPCOM2 register

Parameters:

RegName:

Name of the CAPCOM2 channel to be readed

Macro:

CC2_vPortLock()

Description:

After the call of this macro the contents of the port

register can not changed by the CAPCOM2.

Returnvalue:

None

Parameters:

None

Macro:

CC2_vPortUnlock()

Description:

After the call of this macro the contents of the port

register can changed by the CAPCOM2.

Returnvalue:

None

Parameters:

None

Macro:

CC2_vSelectDoubleRegisterCompareMode(Channel, Mode)

Description:

This macro selects the double register mode for the

selected CAPCOM2 channel. Use this macro only for channels

where the double register compare mode is enabled.

The following definitions are available for Channel:

CC2_CC_24 .. CC2_CC_31

(see @Defines in header file)

The following definitions are available for Mode:

0 - mode is controlled via channel of bank 1

1 - mode is disabled

2 - mode is enabled

Returnvalue:

None

Parameters:

Channel:

Number of the CAPCOM2 channel

Mode:

Double register mode

Macro:

CC2_vSetCCxOUT(Channel)

Description:

This macro sets the output of the selected CAPCOM2 channel

to 1. If the output register is connected to a port pin

then the pin is set to 1 too.

The following definitions are available for Channel (only

for enabled CAPCOM2 channels):

CC2_CC_16 .. CC2_CC_31

(see @Defines in header file)

Returnvalue:

None

Parameters:

Channel:

Name of the CAPCOM2 channel which output should be set

Macro:

CC2_vResetCCxOUT(Channel)

Description:

This macro sets the output of the selected CAPCOM2 channel

to 0. If the output register is connected to a port pin

then the pin is set to 0 too.

The following definitions are available for Channel (only

for enabled CAPCOM2 channels):

CC2_CC_16 .. CC2_CC_31

(see @Defines in header file)

Returnvalue:

None

Parameters:

Channel:

Name of the CAPCOM2 channel which output should be reset

Macro:

CC2_ubGetCCxOUT(Channel)

Description:

This macro returns the output state of selected CAPCOM2

channel.

The following definitions are available for Channel (only

for enabled CAPCOM2 channels):

CC2_CC_16 .. CC2_CC_31

(see @Defines in header file)

Returnvalue:

Current value of the CAPCOM2 output

Parameters:

Channel:

Name of the CAPCOM2 channel which output should be read

Functions:

Function:

void CC2_vInit(void)

Description:

This is the initialization function of the CAPCOM2 function

library. It is assumed that the SFRs used by this library

are in its reset state.

Returnvalue:

None

Parameters:

None

Function:

void CC2_viTmr7(void)

Description:

This is the interrupt service routine for the CAPCOM2 timer

7. It is called when overflow of the timer 7 register

occurs.

Please note that you have to add application specific code

to this function.

Returnvalue:

None

Parameters:

None

Function:

void CC2_viCC28(void)

Description:

This is the interrupt service routine for the CAPCOM2

register CC28. If the content of the corresponding compare

timer (configurable) equals the content of the

capture/compare register CC28 or if a capture event occurs

at the associated port pin, the interrupt request flag is

set and an interrupt is triggered (only if enabled).

Please note that you have to add application specific code

to this function.

Returnvalue:

None

Parameters:

None

Function:

void CC2_viCC30(void)

Description:

This is the interrupt service routine for the CAPCOM2

register CC30. If the content of the corresponding compare

timer (configurable) equals the content of the

capture/compare register CC30 or if a capture event occurs

at the associated port pin, the interrupt request flag is

set and an interrupt is triggered (only if enabled).

Please note that you have to add application specific code

to this function.

Returnvalue:

None

Parameters:

None

Initialization:

Configuration of CAPCOM2 Control:

- the contents of the port register is changed by the CAPCOM2 unit

- staggered mode is disabled

Configuration of CAPCOM2 Timer 7:

- timer 7 works in timer mode

- prescaler factor is 1

- timer 7 run bit is set

Configuration of CAPCOM2 Timer 8:

- timer 8 works in timer mode

- prescaler factor is 1

- timer 8 run bit is reset

Configuration of the used CAPCOM2 Timer Port Pins:

Configuration of the used CAPCOM2 Timer Interrupts:

Tmr7 service request node configuration:

- Tmr7 interrupt priority level (ILVL) = 10

- Tmr7 interrupt group level (GLVL) = 1

- Tmr7 group priority extension (GPX) = 0

Configuration of the used CAPCOM2 Channel 16:

- channel 16 is disabled

Configuration of the used CAPCOM2 Channel 17:

- channel 17 is disabled

Configuration of the used CAPCOM2 Channel 18:

- channel 18 is disabled

Configuration of the used CAPCOM2 Channel 19:

- channel 19 is disabled

Configuration of the used CAPCOM2 Channel 20:

- channel 20 is disabled

Configuration of the used CAPCOM2 Channel 21:

- channel 21 is disabled

Configuration of the used CAPCOM2 Channel 22:

- channel 22 is disabled

Configuration of the used CAPCOM2 Channel 23:

- channel 23 is disabled

Configuration of the used CAPCOM2 Channel 24:

- channel 24 is disabled

Configuration of the used CAPCOM2 Channel 25:

- channel 25 is disabled

Configuration of the used CAPCOM2 Channel 26:

- channel 26 is disabled

Configuration of the used CAPCOM2 Channel 27:

- channel 27 is disabled

Configuration of the used CAPCOM2 Channel 28:

- compare mode 2:  interrupt only

- CC28 allocated to CAPCOM2 timer 7

- single event mode is disabled

Configuration of the used CAPCOM2 Channel 29:

- channel 29 is disabled

Configuration of the used CAPCOM2 Channel 30:

- compare mode 2:  interrupt only

- CC30 allocated to CAPCOM2 timer 7

- single event mode is disabled

Configuration of the used CAPCOM2 Channel 31:

- channel 31 is disabled

Configuration of the used CAPCOM2 Channel Port Pins:

Configuration of the used CAPCOM2 Channels Interrupts:

CC28 service request node configuration:

- CC28 interrupt priority level (ILVL) = 10

- CC28 interrupt group level (GLVL) = 3

- CC28 group priority extension (GPX) = 0

CC30 service request node configuration:

- CC30 interrupt priority level (ILVL) = 10

- CC30 interrupt group level (GLVL) = 2

- CC30 group priority extension (GPX) = 0