For the transmitter to work properly, the user monitor program (which is assumed to be preloaded on the microcontroller and manages the loading and execution of the user program) must allow the transmitter to go in low-power mode (STOP instruction enabled).

I discovered that user monitor described in Motorola application notes prevents the user program to enable the STOP instruction, as it forces the write-once register CONFIG1 to a predefined value. A quick workaround to the problem is to modify the user monitor code from the app notes, setting CONFIG1 register to the following value:

CONFIG1 = bin(0,0,1,0, 0,1,1,1);

//            | | | |  | | | |

//            | | | |  | | | +-COPD    = 1  disable COP watchdog

//            | | | |  | | +---STOP    = 1  enable STOP instruction

//            | | | |  | +-----SSREC   = 1  short (32 cycle) STOP recovery

//            | | | |  +-------LVI5OR3 = 0  set LVI for 3v system

//            | | | +----------LVIPWRD = 0  enable power to LVI system

//            | | +------------LVIRSTD = 1  disable reset on LVI trip

//            | +--------------LVISTOP = 0  disable LVI in STOP mode

//            +----------------COPRS   = 0  long COP timeout

Of course, this procedure applies only to those compiling and loading the program with user-monitor support.