MINI-FLASH-PROGRAMMER MFP51   V1.1

==================================

Disk Contents

=============

- MFP51.EXE    executable program under DOS

- MFP51.DOC    Documentation (this file)

- MFP51.CFG    Configuration file (automatically created)

Program Launch:

===================

MFP51        Starts the programmer's user interface.

MFP51 /auto  Starts the command line version.

Brief program description

=========================

SELECT       : 89C1051  Selects the uController used.

89C2051  89C1051 (1k ROM) or 89C2051 (2k ROM)

READ         : on screen: the contents of the uC is

displayed on the screen in blocks

of 256 bytes.

The display format is Hex and ASCII.

to file:    the contents of the uC is read

and copied into a file

blankcheck: All memory cells of the uC are

checked for $FF.

identifier: The signature bytes of the uC

fitted in the circuit are read.

verify:     The contents of the uC are compared

against a file.

WRITE   :   From file: the contents of a file are

written into the uC.

Lockbit 1: Lockbit 1 of the uC is set.

Disables re-programming of the

chip

Lockbit 1+2: Lockbits 1+2 of the uC are set.

Disables re-programming and

reading of the chip.

Flash clear: The Flash range of the uC is

cleared.

All memory cells are cleared with

value $FF.

CONFIG  : German    : German text output

English   : English text output

LPT1      : Connection to parallel interface LPT1

LPT2      : Connection to parallel interface LPT2

< ESC >         : Quits the program

All user selections are stored (MFP51.CFG) and are available

when the program is called again.

Command Line version

====================

A command line version of the program is available for

automated program sequences. This version may be launched by a

Batch file or similar.

Launch: MFP51 /auto

Note: The programmer works with the settings stored in the

configuration file MFP51.CFG. So make sure beforehand whether

the name of the source file, the uC type, etc, is properly

set.

Also ensure that the programmer is properly connected, and

that a chip is fitted. Although a programming error which may

occur is detected, it is only reported with the global

message 'error'.

An investigation into the cause of the problem should then be

started with the aid of the main program.