# Appendix C: USB Keyboard/Keypad Page (0x07)

This section is the Usage Page for key codes to be used in implementing a USB keyboard. A Boot Keyboard (84-, 101- or 104-key) should at a minimum support all associated usage codes as indicated in the “Boot” column below.

Note:  Due to the variation of keyboards from language to language, it is not feasible to specify exact key mappings for every language. Where this list is not specific for a key function in a language, the closest equivalent key position should be used, so that a keyboard may be modified for a different language by simply printing different keycaps. One example is the Y key on a North American keyboard. In Germany, this is typically Z. Rather than changing the keyboard firmware to put the Z Usage into that place in the descriptor list, the vendor should use the Y Usage on both the North American and German keyboards. This continues to be the existing practice in the industry, in order to minimize the number of changes to the electronics to accommodate other languages.

| Usage index (dec) | Usage Index (hex) |  | Usage | Ref:typical AT-101 position | PC-AT | Mac- intosh |  | UNIX | Boot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 00 |  | Reserved (no event indicated) 9 | N/A |  |  |  |  | 84/101/104 |
| 1 | 01 |  | Keyboard ErrorRollOver9 | N/A |  |  |  |  | 84/101/104 |
| 2 | 02 |  | Keyboard POSTFail9 | N/A |  |  |  |  | 84/101/104 |
| 3 | 03 |  | Keyboard ErrorUndefined9 | N/A |  |  |  |  | 84/101/104 |
| 4 | 04 |  | Keyboard a and A4 | 31 |  |  |  |  | 84/101/104 |
| 5 | 05 |  | Keyboard b and B | 50 |  |  |  |  | 84/101/104 |
| 6 | 06 |  | Keyboard c and C4 | 48 |  |  |  |  | 84/101/104 |
| 7 | 07 |  | Keyboard d and D | 33 |  |  |  |  | 84/101/104 |
| 8 | 08 |  | Keyboard e and E | 19 |  |  |  |  | 84/101/104 |
| 9 | 09 |  | Keyboard f and F | 34 |  |  |  |  | 84/101/104 |
| 10 | 0A |  | Keyboard g and G | 35 |  |  |  |  | 84/101/104 |
| 11 | 0B |  | Keyboard h and H | 36 |  |  |  |  | 84/101/104 |
| 12 | 0C |  | Keyboard i and I | 24 |  |  |  |  | 84/101/104 |
| 13 | 0D |  | Keyboard j and J | 37 |  |  |  |  | 84/101/104 |
| 14 | 0E |  | Keyboard k and K | 38 |  |  |  |  | 84/101/104 |
| 15 | 0F |  | Keyboard l and L | 39 |  |  |  |  | 84/101/104 |
| 16 | 10 |  | Keyboard m and M4 | 52 |  |  |  |  | 84/101/104 |
| 17 | 11 |  | Keyboard n and N | 51 |  |  |  |  | 84/101/104 |
| 18 | 12 |  | Keyboard o and O4 | 25 |  |  |  |  | 84/101/104 |
| 19 | 13 |  | Keyboard p and P4 | 26 |  |  |  |  | 84/101/104 |
| 20 | 14 |  | Keyboard q and Q4 | 17 |  |  |  |  | 84/101/104 |
| 21 | 15 |  | Keyboard r and R | 20 |  |  |  |  | 84/101/104 |
| 22 | 16 |  | Keyboard s and S4 | 32 |  |  |  |  | 84/101/104 |
| 23 | 17 |  | Keyboard t and T | 21 |  |  |  |  | 84/101/104 |
| 24 | 18 |  | Keyboard u and U | 23 |  |  |  |  | 84/101/104 |
| 25 | 19 |  | Keyboard v and V | 49 |  |  |  |  | 84/101/104 |
| 26 | 1A |  | Keyboard w and W4 | 18 |  |  |  |  | 84/101/104 |
| 27 | 1B |  | Keyboard x and X4 | 47 |  |  |  |  | 84/101/104 |
| 28 | 1C |  | Keyboard y and Y4 | 22 |  |  |  |  | 84/101/104 |
| 29 | 1D |  | Keyboard z and Z4 | 46 |  |  |  |  | 84/101/104 |
| 30 | 1E |  | Keyboard 1 and ! 4 | 2 |  |  |  |  | 84/101/104 |
| 31 | 1F |  | Keyboard 2 and @4 | 3 |  |  |  |  | 84/101/104 |
| 32 | 20 |  | Keyboard 3 and #4 | 4 |  |  |  |  | 84/101/104 |
| 33 | 21 |  | Keyboard 4 and $4 | 5 |  |  |  |  | 84/101/104 |
| 34 | 22 |  | Keyboard 5 and %4 | 6 |  |  |  |  | 84/101/104 |
| 35 | 23 |  | Keyboard 6 and ^4 | 7 |  |  |  |  | 84/101/104 |
| 36 | 24 |  | Keyboard 7 and &4 | 8 |  |  |  |  | 84/101/104 |
| 37 | 25 |  | Keyboard 8 and *4 | 9 |  |  |  |  | 84/101/104 |
| 38 | 26 |  | Keyboard 9 and (4 | 10 |  |  |  |  | 84/101/104 |
| 39 | 27 |  | Keyboard 0 and ) 4 | 11 |  |  |  |  | 84/101/104 |
| 40 | 28 |  | Keyboard Return(ENTER) 5 | 43 |  |  |  |  | 84/101/104 |
| 41 | 29 |  | Keyboard ESCAPE | 110 |  |  |  |  | 84/101/104 |
| 42 | 2A |  | Keyboard DELETE (Backspace) 13 | 15 |  |  |  |  | 84/101/104 |
| 43 | 2B |  | Keyboard Tab | 16 |  |  |  |  | 84/101/104 |
| 44 | 2C |  | Keyboard Spacebar | 61 |  |  |  |  | 84/101/104 |
| 45 | 2D |  | Keyboard - and (underscore) 4 | 12 |  |  |  |  | 84/101/104 |
| 46 | 2E |  | Keyboard = and+4 | 13 |  |  |  |  | 84/101/104 |
| 47 | 2F |  | Keyboard [ and {4 | 27 |  |  |  |  | 84/101/104 |
| 48 | 30 |  | Keyboard ] and }4 | 28 |  |  |  |  | 84/101/104 |
| 49 | 31 |  | Keyboard \ and ¦ | 29 |  |  |  |  | 84/101/104 |
| 50 | 32 |  | Keyboard Non-US# and ~2 | 42 |  |  |  |  | 84/101/104 |
| 51 | 33 |  | Keyboard 4 | 40 |  |  |  |  | 84/101/104 |
| 52 | 34 |  | Keyboard ‘ and “4 | 41 |  |  |  |  | 84/101/104 |
| 53 | 35 |  | Keyboard Grave Accent and Tilde4 | 1 |  |  |  |  | 84/101/104 |
| 54 | 36 |  | Keyboard , and <4 | 53 |  |  |  |  | 84/101/104 |
| 55 | 37 |  | Keyboard . and >4 | 54 |  |  |  |  | 84/101/104 |
| 56 | 38 |  | Keyboard / and ? 4 | 55 |  |  |  |  | 84/101/104 |
| 57 | 39 |  | Keyboard CapsLock11 | 30 |  |  |  |  | 84/101/104 |
| 58 | 3A |  | Keyboard F1 | 112 |  |  |  |  | 84/101/104 |
| 59 | 3B |  | Keyboard F2 | 113 |  |  |  |  | 84/101/104 |
| 60 | 3C |  | Keyboard F3 | 114 |  |  |  |  | 84/101/104 |
| 61 | 3D |  | Keyboard F4 | 115 |  |  |  |  | 84/101/104 |
| 62 | 3E |  | Keyboard F5 | 116 |  |  |  |  | 84/101/104 |
| 63 | 3F |  | Keyboard F6 | 117 |  |  |  |  | 84/101/104 |
| 64 | 40 |  | Keyboard F7 | 118 |  |  |  |  | 84/101/104 |
| 65 | 41 |  | Keyboard F8 | 119 |  |  |  |  | 84/101/104 |
| 66 | 42 |  | Keyboard F9 | 120 |  |  |  |  | 84/101/104 |
| 67 | 43 |  | Keyboard F10 | 121 |  |  |  |  | 84/101/104 |
| 68 | 44 |  | Keyboard F11 | 122 |  |  |  |  | 101/104 |
| 69 | 45 |  | Keyboard F12 | 123 |  |  |  |  | 101/104 |
| 70 | 46 |  | Keyboard PrintScreen1 | 124 |  |  |  |  | 101/104 |
| 71 | 47 |  | Keyboard ScrollLock11 | 125 |  |  |  |  | 84/101/104 |
| 72 | 48 |  | Keyboard Pause1 | 126 |  |  |  |  | 101/104 |
| 73 | 49 |  | Keyboard Insert1 | 75 |  |  |  |  | 101/104 |
| 74 | 4A |  | Keyboard Home1 | 80 |  |  |  |  | 101/104 |
| 75 | 4B |  | Keyboard PageUp1 | 85 |  |  |  |  | 101/104 |
| 76 | 4C |  | Keyboard Delete Forward1 | 76 |  |  |  |  | 101/104 |
| 77 | 4D |  | Keyboard End1 | 81 |  |  |  |  | 101/104 |
| 78 | 4E |  | Keyboard PageDown1 | 86 |  |  |  |  | 101/104 |
| 79 | 4F |  | Keyboard RightArrow1 | 89 |  |  |  |  | 101/104 |
| 80 | 50 |  | Keyboard LeftArrow1 | 79 |  |  |  |  | 101/104 |
| 81 | 51 |  | Keyboard DownArrow1 | 84 |  |  |  |  | 101/104 |
| 82 | 52 |  | Keyboard UpArrow1 | 83 |  |  |  |  | 101/104 |
| 83 | 53 |  | Keypad NumLock and Clear11 | 90 |  |  |  |  | 101/104 |
| 84 | 54 |  | Keypad /1 | 95 |  |  |  |  | 101/104 |
| 85 | 55 |  | Keypad * | 100 |  |  |  |  | 84/101/104 |
| 86 | 56 |  | Keypad - | 105 |  |  |  |  | 84/101/104 |
| 87 | 57 |  | Keypad + | 106 |  |  |  |  | 84/101/104 |
| 88 | 58 |  | Keypad ENTER5 | 108 |  |  |  |  | 101/104 |
| 89 | 59 |  | Keypad 1 and End | 93 |  |  |  |  | 84/101/104 |
| 90 | 5A |  | Keypad 2 and Down Arrow | 98 |  |  |  |  | 84/101/104 |
| 91 | 5B |  | Keypad 3 and PageDn | 103 |  |  |  |  | 84/101/104 |
| 92 | 5C |  | Keypad 4 and Left Arrow | 92 |  |  |  |  | 84/101/104 |
| 93 | 5D |  | Keypad 5 | 97 |  |  |  |  | 84/101/104 |
| 94 | 5E |  | Keypad 6 and Righ tArrow | 102 |  |  |  |  | 84/101/104 |
| 95 | 5F |  | Keypad 7 and Home | 91 |  |  |  |  | 84/101/104 |
| 96 | 60 |  | Keypad 8 and Up Arrow | 96 |  |  |  |  | 84/101/104 |
| 97 | 61 |  | Keypad 9 and PageUp | 101 |  |  |  |  | 84/101/104 |
| 98 | 62 |  | Keypad 0 and Insert | 99 |  |  |  |  | 84/101/104 |
| 99 | 63 |  | Keypad . and Delete | 104 |  |  |  |  | 84/101/104 |
| 100 | 64 |  | Keyboard Non-US\ and ¦3;6 | 45 |  |  |  |  | 84/101/104 |
| 101 | 65 |  | Keyboard Application10 | 129 |  |  |  |  | 104 |
| 102 | 66 |  | Keyboard Power9 |  |  |  |  |  |  |
| 103 | 67 |  | Keypad = |  |  |  |  |  |  |
| 104 | 68 |  | Keyboard F13 |  |  |  |  |  |  |
| 105 | 69 |  | Keyboard F14 |  |  |  |  |  |  |
| 106 | 6A |  | Keyboard F15 |  |  |  |  |  |  |
| 107 | 6B |  | Keyboard F16 |  |  |  |  |  |  |
| 108 | 6C |  | Keyboard F17 |  |  |  |  |  |  |
| 109 | 6D |  | Keyboard F18 |  |  |  |  |  |  |
| 110 | 6E |  | Keyboard F19 |  |  |  |  |  |  |
| 111 | 6F |  | Keyboard F20 |  |  |  |  |  |  |
| 112 | 70 |  | Keyboard F21 |  |  |  |  |  |  |
| 113 | 71 |  | Keyboard F22 |  |  |  |  |  |  |
| 114 | 72 |  | Keyboard F23 |  |  |  |  |  |  |
| 115 | 73 |  | Keyboard F24 |  |  |  |  |  |  |
| 116 | 74 |  | Keyboard Execute |  |  |  |  |  |  |
| 117 | 75 |  | Keyboard Help |  |  |  |  |  |  |
| 118 | 76 |  | Keyboard Menu |  |  |  |  |  |  |
| 119 | 77 |  | Keyboard Select |  |  |  |  |  |  |
| 120 | 78 |  | Keyboard Stop |  |  |  |  |  |  |
| 121 | 79 |  | Keyboard Again |  |  |  |  |  |  |
| 122 | 7A |  | Keyboard Undo |  |  |  |  |  |  |
| 123 | 7B |  | Keyboard Cut |  |  |  |  |  |  |
| 124 | 7C |  | Keyboard Copy |  |  |  |  |  |  |
| 125 | 7D |  | Keyboard Paste |  |  |  |  |  |  |
| 126 | 7E |  | Keyboard Find |  |  |  |  |  |  |
| 127 | 7F |  | Keyboard Mute |  |  |  |  |  |  |
| 128 | 80 |  | Keyboard Volume Up |  |  |  |  |  |  |
| 129 | 81 |  | Keyboard Volume Down |  |  |  |  |  |  |
| 130 | 82 |  | Keyboard Locking Caps Lock12 |  |  |  |  |  |  |
| 131 | 83 |  | Keyboard Locking Num Lock12 |  |  |  |  |  |  |
| 132 | 84 |  | Keyboard Locking Scroll |  |  |  |  |  |  |
|  |  |  | Lock 12 |  |  |  |  |  |  |
| 133 | 85 |  | Keypad Comma |  |  |  |  |  |  |
| 134 | 86 |  | Keypad Equal Sign |  |  |  |  |  |  |
| 135 | 87 |  | Keyboard Kanji115 |  |  |  |  |  |  |
| 136 | 88 |  | Keyboard Kanji216 |  |  |  |  |  |  |
| 137 | 89 |  | Keyboard Kanji317 |  |  |  |  |  |  |
| 138 | 8A |  | Keyboard Kanji418 |  |  |  |  |  |  |
| 139 | 8B |  | Keyboard Kanji519 |  |  |  |  |  |  |
| 140 | 8C |  | Keyboard Kanji620 |  |  |  |  |  |  |
| 141 | 8D |  | Keyboard Kanji721 |  |  |  |  |  |  |
| 142 | 8E |  | Keyboard Kanji822 |  |  |  |  |  |  |
| 143 | 8F |  | Keyboard Kanji922 |  |  |  |  |  |  |
| 144 | 90 |  | Keyboard LANG18 |  |  |  |  |  |  |
| 145 | 91 |  | Keyboard LANG28 |  |  |  |  |  |  |
| 146 | 92 |  | Keyboard LANG38 |  |  |  |  |  |  |
| 147 | 93 |  | Keyboard LANG48 |  |  |  |  |  |  |
| 148 | 94 |  | Keyboard LANG58 |  |  |  |  |  |  |
| 149 | 95 |  | Keyboard LANG68 |  |  |  |  |  |  |
| 150 | 96 |  | Keyboard LANG78 |  |  |  |  |  |  |
| 151 | 97 |  | Keyboard LANG88 |  |  |  |  |  |  |
| 152 | 98 |  | Keyboard LANG98 |  |  |  |  |  |  |
| 153 | 99 |  | Keyboard AlternateErase7 |  |  |  |  |  |  |
| 154 | 9A |  | Keyboard SysReq/Attenti1 |  |  |  |  |  |  |
| 155 | 9B |  | Keyboard Cancel |  |  |  |  |  |  |
| 156 | 9C |  | Keyboard Clear |  |  |  |  |  |  |
| 157 | 9D |  | Keyboard Prior |  |  |  |  |  |  |
| 158 | 9E |  | Keyboard Return |  |  |  |  |  |  |
| 159 | 9F |  | Keyboard Separator |  |  |  |  |  |  |
| 160 | A0 |  | Keyboard Out |  |  |  |  |  |  |
| 161 | A1 |  | Keyboard Oper |  |  |  |  |  |  |
| 162 | A2 |  | Keyboard Clear/Again |  |  |  |  |  |  |
| 163 | A3 |  | Keyboard CrSel/Props |  |  |  |  |  |  |
| 164 | A4 |  | Keyboard ExSel |  |  |  |  |  |  |
| 165-223 | A5-DF |  | Reserved |  |  |  |  |  |  |
| 224 | E0 |  | Keyboard LeftControl | 58 |  |  |  |  | 84/101/104 |
| 225 | E1 |  | Keyboard LeftShift | 44 |  |  |  |  | 84/101/104 |
| 226 | E2 |  | Keyboard LeftAlt | 60 |  |  |  |  | 84/101/104 |
| 227 | E3 |  | Keyboard Left GUI10;23 | 127 |  |  |  |  | 104 |
| 228 | E4 |  | Keyboard RightControl | 64 |  |  |  |  | 101/104 |
| 229 | E5 |  | Keyboard RightShift | 57 |  |  |  |  | 84/101/104 |
| 230 | E6 |  | Keyboard RightAlt | 62 |  |  |  |  | 101/104 |
| 231 | E7 |  | Keyboard Right GUI10;24 | 128 |  |  |  |  | 104 |
| 232-255 | E8-FF | Reserved |  |  |  |  |  |  |  |