Bus 001 Device 009: ID 03eb:2140 Atmel Corp.

Device Descriptor:

bLength                18

bDescriptorType         1

bcdUSB               2.00

bDeviceClass          239 Miscellaneous Device

bDeviceSubClass         2 ?

bDeviceProtocol         1 Interface Association

bMaxPacketSize0        64

idVendor           0x03eb Atmel Corp.

idProduct          0x2140

bcdDevice            1.01

iManufacturer           1 Atmel Corp.

iProduct                2 JTAGICE3 CMSIS-DAP

iSerial                 3 J30200010645

bNumConfigurations      1

Configuration Descriptor:

bLength                 9

bDescriptorType         2

wTotalLength           64

bNumInterfaces          2

bConfigurationValue     1

iConfiguration          0

bmAttributes         0x80

(Bus Powered)

MaxPower              100mA

Interface Descriptor:

bLength                 9

bDescriptorType         4

bInterfaceNumber        0

bAlternateSetting       0

bNumEndpoints           2

bInterfaceClass         3 Human Interface Device

bInterfaceSubClass      0 No Subclass

bInterfaceProtocol      0 None

iInterface              4 JTAGICE3 CMSIS-DAP

HID Device Descriptor:

bLength                 9

bDescriptorType        33

bcdHID               1.11

bCountryCode            0 Not supported

bNumDescriptors         1

bDescriptorType        34 Report

wDescriptorLength      35

Report Descriptor: (length is 35)

Item(Global): Usage Page, data= [ 0x00 0xff ] 65280

(null)

Item(Local ): Usage, data= [ 0x01 ] 1

(null)

Item(Main  ): Collection, data= [ 0x01 ] 1

Application

Item(Global): Logical Minimum, data= [ 0x00 ] 0

Item(Global): Logical Maximum, data= [ 0xff 0x00 ] 255

Item(Global): Report Size, data= [ 0x08 ] 8

Item(Global): Report Count, data= [ 0x00 0x02 ] 512

Item(Local ): Usage, data= [ 0x01 ] 1

(null)

Item(Main  ): Input, data= [ 0x02 ] 2

Data Variable Absolute No_Wrap Linear

Preferred_State No_Null_Position Non_Volatile Bitfield

Item(Global): Report Count, data= [ 0x00 0x02 ] 512

Item(Local ): Usage, data= [ 0x01 ] 1

(null)

Item(Main  ): Output, data= [ 0x02 ] 2

Data Variable Absolute No_Wrap Linear

Preferred_State No_Null_Position Non_Volatile Bitfield

Item(Global): Report Count, data= [ 0x04 ] 4

Item(Local ): Usage, data= [ 0x01 ] 1

(null)

Item(Main  ): Feature, data= [ 0x02 ] 2

Data Variable Absolute No_Wrap Linear

Preferred_State No_Null_Position Non_Volatile Bitfield

Item(Main  ): End Collection, data=none

Endpoint Descriptor:

bLength                 7

bDescriptorType         5

bEndpointAddress     0x01  EP 1 OUT

bmAttributes            3

Transfer Type            Interrupt

Synch Type               None

Usage Type               Data

wMaxPacketSize     0x0200  1x 512 bytes

bInterval               1

Endpoint Descriptor:

bLength                 7

bDescriptorType         5

bEndpointAddress     0x82  EP 2 IN

bmAttributes            3

Transfer Type            Interrupt

Synch Type               None

Usage Type               Data

wMaxPacketSize     0x0200  1x 512 bytes

bInterval               1

Interface Descriptor:

bLength                 9

bDescriptorType         4

bInterfaceNumber        1

bAlternateSetting       0

bNumEndpoints           2

bInterfaceClass       255 Vendor Specific Class

bInterfaceSubClass    255 Vendor Specific Subclass

bInterfaceProtocol    255 Vendor Specific Protocol

iInterface              5 JTAGICE3 Data Gateway

Endpoint Descriptor:

bLength                 7

bDescriptorType         5

bEndpointAddress     0x84  EP 4 IN

bmAttributes            2

Transfer Type            Bulk

Synch Type               None

Usage Type               Data

wMaxPacketSize     0x0200  1x 512 bytes

bInterval             255

Endpoint Descriptor:

bLength                 7

bDescriptorType         5

bEndpointAddress     0x03  EP 3 OUT

bmAttributes            2

Transfer Type            Bulk

Synch Type               None

Usage Type               Data

wMaxPacketSize     0x0200  1x 512 bytes

bInterval             255

Device Qualifier (for other device speed):

bLength                10

bDescriptorType         6

bcdUSB               2.00

bDeviceClass          239 Miscellaneous Device

bDeviceSubClass         2 ?

bDeviceProtocol         1 Interface Association

bMaxPacketSize0        64

bNumConfigurations      1

Device Status:     0x0000

(Bus Powered)