Start with ESP32-CAM

I use Arduino IDE test my new toy, an ESP32-CAM, now I’ll show you how to start with it.

First of all, we need get ready our hardware：

- An ESP32-CAM dev board

- A camera, like OV2640 or OV7670, I use the OV2640 for test

- Ps:I think the OV2640 for test, the OV7670 better for projects

- 3、TTL to USB module

- Next step, software part:

- Arduino IDE(https://www.arduino.cc/en/software/)

- There are Arduino IDE for Windows, for Linux and for Mac OS

- Step 3

- Put your ESP32-CAM link to TTL to USB module

- TTL to USB 5V link ESP32-CAM 5V

- TTL to USB GND link ESP32-CAM GND

- TTL to USB TXD link ESP32-CAM U0R

- TTL to USB RXD link ESP32-CAM U0T

- Like this:

- Step 4

- Take your TTL to USB module into USB interface of your computer, and open your PC Manage——Device Manager——Ports, remember your COM

- Step 5

- Open your Arduino IDE, File——Preferences, take https://dl.espressif.com/dl/package_esp32_index.json into Additional Boards Manager URLs

- Step 6

- Click Tools, find Board, choose AI Thinker ESP32-CAM, find CPU Frequency, choose 160MHz(WiFi/BT), find Port, choose your own COM(in Step 4)

- Step 7

- Click File——Examples——WiFi——WiFiScan

- Step 8

- Remove // in front of #define CAMERA_MODEL_AI_THINKER

- Step 9

- Put your ssid and password into here

- Step 10

- Link IO0 and GND of your ESP32-CAM(important)

- Upload the code to your ESP32-CAM dev board

- Remove IO0 to GND jumper wire

- Open Tools——Serial Monitor, you can find a IP address, copy it and paste in your browser

- You can use your camera now, if you want Face Detection, Resolution must be CIF(400x296)

- Enjoy it and share it to your friends