GP36-S5050-RGB

Features:                                    Attention:

5000MM*14MM*7MM WATERPROOF LED MOD      OBSERVE PRECAUTIONS WORKING BOLTAGE: 12 VDC                       ELECTROSTATIC

WORKING CURRENT:3600MA                       DISCHARGE

LED VIEW ANGLE(2θ1/2)：140                       SENSITIVE

WATER PROOF: IP67                                DEVICES

BUILD IN PWM SIGNAL GENERATOR IC              FOR HANDLING

COLOR CHANGES

Package Dimensions

Notes :  1. All dimension units are millimeters.

2. All dimension tolerance is ±1mm unless otherwise noted

Connection diagram

| Parameter | Symbol | Absolute Maximum Rating | Unit |
| --- | --- | --- | --- |
| Forward Current | IF | 3600 | mA |
| Power Dissipation | PD | 43.2 | W |
| Electrostatic discharge | ESD | 1000 | V |
| Operating Temperature | Topr | -25∽+60 | ℃ |
| Storage Temperature | Tstg | -40∽+80 | ℃ |

| Parameter | Symbol | Condition |  | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Forward Voltage DC | VF | IF=3600mA |  | --- | 12 | --- | V |
| Dominant Wavelength | λD | IF=3600mA | R | 620- |  | 625 | nm |
|  |  |  | G | 515 |  | 520 | nm |
|  |  |  | B | 465 |  | 470 | nm |
| Luminous flux | φV | IF=3600mA |  | --- | --- | --- | lm |
| Recommend Forward Current | IF(rec) |  |  | --- | 3600 | --- | mA |