Speed Controller Programming Instructions

For ESC with simple options

30A

30A :  BEC:3A   2-3LIPO/5-10NIMH   MAX:45A/12S

Programming Instructions:

- Connect your motor and receiver to the speed controller, but do not connect the battery yet.

2. Turn on your transmitter and full up the throttle stick. Please Note: Most Futaba transmitters have the throttle channel reversed by default.

3. Connect your battery and you will hear a starting beep tone. After 3 seconds, the controller will start beeping a sequence as following. All beeps only repeat once.

4. To select, move the throttle stick in off position, between Beeps repeat. Then you will hear a confirmation sound.

5. Each time when you select an option, it will exit programming menu. You may restart program again.

Options Table (for Aircraft version Normal mode):

|  | Beeping | Options |  |
| --- | --- | --- | --- |
| 1 | －，－ | Cell Type: LIPO | ESC identify the number of LIPO and set LIPO protection automatically Cut-off voltage at 2.9V per Lipo (option Two will be disable automatically) |
| 2 | ――,―― | Cell Type: NiMH | Select NIMH protection mode Cut-off voltage at 60% of starting voltage (option ONE will be disable automatically) |
| 3 | ―――，―――， | Break Enable/ Disable | Break enabled by Default |